DUAL SHOCK

SCEE Technical Note

June 1998

# Key Differences.

This is a list of the main differences between the Analog Controller (DUAL SHOCK) and the original Analog Controller.

- The addition of a vibration function

- The removal of the "Green" mode featured on the original Analog Controller

- Improved L3/R3 button to allow greater grip.

## Basics.

There are 2 motors contained within the Controller, A small motor in the Right handle and a large motor in the left handle.

- The larger of the 2 motors provides a significant jolt when at full power. The vibration can be changed between 0-255, 0 being off and 255 being maximum power.

- The small motor only has 2 settings 0-1, 0 being off and 1 being on.

## Advanced.

The limits of the motors are not as constrained as they may seem as it is possible to pulse the motors to gain a large range of sensations.  Please see sample code fragments below.  This code is also available on the Sony Developer Website.

# Sample Code

This sample code displays the Pulsing technique mentioned above.  It displays the state of the motor at every frame along with the DUAL SHOCK stick data.

#define 	PORT1 	0x00

/* Controller Information buffer */

static unsigned char padbuff[2][34];

static unsigned char align[6]={0,1,0xFF,0xFF,0xFF,0xFF};

static struct {

unsigned char Button;

unsigned char Lock;

unsigned char Motor0,Motor1;

unsigned char Send;

} hist;

int main( void )

{

unsigned char* padi;

unsigned char* pad2;

int MotorState0,MotorState1;

PadInitDirect(padbuff[0],padbuff[1]);

PadStartCom();

hist.Button = 0;

hist.Lock = 0;

hist.Motor0 = 0;

hist.Motor1 = 0;

hist.Send = 0;

ids = limitt = state = 0;

cdb  = (cdb==db)? db+1: db;

while(1)

{

padi = padbuff[0];

pad2 = padbuff[1];

state = PadGetState(PORT1);

cdb  = (cdb==db)? db+1: db;

if (state==PadStateDiscon) {

FntPrint("NO CONTROLLER\n" );

}

else

{

switch((ids=PadInfoMode(PORT1,InfoModeCurID,0))) {

/* Analog Controller */

case 7:

hist.Lock = 1;

FntPrint("\n\t\tHex Values of L3 [%x][%x]\n\t\tHex Values of R3 [%x][%x]",padi[7]

,padi[6] ,padi[5],padi[4]);

FntPrint("\n\t\tMotor 0 = %d",MotorState0);

FntPrint("\n\t\tMotor 1 = %d",MotorState1);

if (PadInfoMode(0x00,InfoModeCurExID,0)) {

// Alters Motor0 with the data from L3

n = abs(triming(padi[5]))|(abs(triming(padi[4])) ? 20:0);

if ( n>0x60) n = 0x60;

limitt += n;

if (limitt>=0x60) {

hist.Motor0 = 1;

MotorState0 = 1;

limitt -= 0x60;

}

else

{

hist.Motor0 = 0;

MotorState0 = 0;

}

// Alters Motor1 with the data from R3

n = abs(triming(padi[7]))|(abs(triming(padi[6])) ? 20:0);

if ( n>0x60 ) n = 0x60;

limitt2 += n;

if (limitt2>=0x60) {

hist.Motor1 = 255;

MotorState1 = 1;

limitt2 -= 0x60;

}

else

{

MotorState1 = 0;

hist.Motor1 = 0;

}

break;

// if not the analog controller then set the moters to 0.

default:

hist.Motor0 = 0;

hist.Motor1 = 0;

FntPrint("\nPlease Insert  an Analog Controller (DUAL SHOCK) or make sure \

that the Controller is in Analog Controller (DUAL SHOCK) Mode");

break;

}

}

}

if (state == PadStateFindPad) {

hist.Send = 0;

}

if ( hist.Send==0 ) {

PadSetAct(0x00,&(hist.Motor0),2);

if (state == PadStateFindCTP1) {

hist.Send = 1;

}

if (state == PadStateStable) {

if (PadSetActAlign(0x00,align)) {

hist.Send = 1;

}

}

}

PadSetAct(0x00, &hist.Motor0, 2);

VSync(0);

PutDispEnv(&cdb->disp);

PutDrawEnv(&cdb->draw);

FntFlush(-1);

}

DrawSync(0);

PadStop();

}

long triming( unsigned char pad )

{

long n;

n = 0;

if (pad<=0x60 ) n = (0x60-pad);

if (pad>=0xa0 ) n = (0x9f-pad);

return(n);

}

# QA and Naming Conventions.

Please see the new TRC v.1.3 page 13 for all naming conventions. Also the following list may be used as a quick reference and also covers items not in the TRC

Manual Usage:

1) Analog Controller (DUAL SHOCK)

2) Analog Controller (Dual Shock)

3) Analog Controller

In Game Usage (possible space constraints)

1) Analog Controller (DUAL SHOCK)

2) Analog Controller (Dual Shock)

3) Analog Controller

4) DUAL SHOCK (Only if space does not allow full name)

5) Dual Shock (Only if space does not allow full name)

'Dual Shock' Mode

Manual Usage:

1) Vibration Mode

In Game Usage (possible space constraints)

1) Vibration Mode

2) Vibration

In titles that support Analog Controller (DUAL SHOCK) the code should have an option called 'Vibration ON/OFF'

Language Skews

Example: Manette Analogique ( Dual Shock )

# Usage.

There are a number of uses that the Analog Controller (DUAL SHOCK) could be used for.  And here are a few Ideas:

- To simulate the vibration experienced when driving.

- To give a feeling of contact during a beat'em'up

- Pulse the Analog Controller (DUAL SHOCK) when in a football game to simulate the feeling of running and kicking the ball.  The faster you are running the more severe the Pulse should be.

All of these ideas should be very easy to implement with the sample code provided.

# FAQ'S

Q. When using the Analog Controller (DUAL SHOCK) with a Multi Tap, how many controllers can vibrate at the same time, is it linked to the intensity of the vibration or is there a single figure?

###### A. The total drain via the PlayStation cannot be more than 60 currency units, the following table shows the drain per motor.

| ###### Model number | ###### Actuator type | ###### Currency drain |
| --- | --- | --- |
| ###### SCPH-1150 | ###### actuator 0 | ###### 10 units |
| ###### SCPH-1200 | ###### actuator 0 | ###### 10 units |
| ###### SCPH-1200 | ###### actuator 1 | ###### 20 units |

###### If total current drain exceeds 60 units, the library will ignore any other vibration requests. Therefore it is possible to vibrate any combination of controllers and motors’ providing this currency drain does not exceed 60 units.

###### This can be demonstrated using the source code in the multitap directory. Which also shows correct detection of the Dual Shock and other controllers, as well as correct initialisation.

###### One solution which can give the illusion that the vibration is exceeding 60 units would be to stagger the vibrations each frame between controllers like so

frame count  |  vibrating controller

|    1 2 3 4 5 6 7 8

-----------------+-----------------------

0             |    o o x x x x x x

1             |    x x o o x x x x

2             |    x x x x o o x x

3             |    x x x x x x o o

4             |    o o x x x x x x

...             |          ...

Q) Why does my controller Become Unrecognised?

A. The reference that came with the 4.3 libraries included this comment ..

“When an Analog Controller (DUAL SHOCK) was used, if PadStartCom()

was executed within three seconds after PadStopCom(), the Analog

Controller (DUAL SHOCK) became unable to be recognised.  This problem

has been fixed.”

So upgrade to the 4.3 libraries.

Q) Why does the Dual Shock ‘drop out’ of Analog Mode?

A. This problem is dealt with in detail within the documents and source code in the dropout directory that this document came with. (See readme.txt in the dropout directory)

Q) Help, my Dual Shock stops rumbling if I switch modes frequently.

A. The Dual Shock can sometimes fail to load actuator information, this is actually done by the PadSetActAlign() command. Check that the return value is 1 for success. If the command fails, wait about a tenth of a second and call the function again.

Q) Argh, I’ve just tried to put libpad.lib into my project to support the Dual Shock and it’s intermittent and my memory card system has stopped working.

A. Using libpad.lib requires a different initialisation sequence to the standard libraries. This was done just to keep you on your toes. Please refer to the following startup code when using libpad.

1.8 Usage with memory card

When using memory card with this library, initialize memory card prior

to controller initialization. Also specify arguments to 0 for calling

InitCARD(). And call PadInitDirect() (or PadInitMtap()) before calling

PadStartCom().

InitCARD(0);

StartCARD();

_bu_init();

PadInitDirect();

PadSetAct(...);

PadStartCom();

Make sure that the arguments of MemCardInit() are also set to "0" when

High-level memory card library (libmcrd) is used.  Call MemCardInit(0)

and MemCardStart() before initializing controller.

MemCardInit(0);

MemCardStart();

PadInitDirect();

PadSetAct(...);

PadStartCom();

## Notes.

The Reference code was the Dungeon code from SCEI.  Please see the lib4.3 sample code on the developers web site.

If you have any problems with the Analog Controller (DUAL SHOCK ) Do not hesitate to contact SCEE Developer Support.