Performance Analyzer

Technical Manual

July1997

This manual is distributed to “ PlayStation” licensed developers. Information in this manual is subject

to change without notice. Unauthorized reproduction, distribution or disclosure to third parties of the

whole or any part of this manual, in any form or by any means, for any purpose, is expressly

prohibited by the License Developer Agreement.

“ PlayStation” is a registered trademark of Sony Computer Entertainment Inc.

Company and product names recorded in/on this product are generally trademarks of each company.

Note that in/on this product the symbols “Ò” and “Ô” are not used explicitly.

Published July 1997

© 1997 Sony Computer Entertainment Inc. All Rights Reserved.

Written and produced by:

Sony Computer Entertainment Inc.

7-1-1 Akasaka, Minato-ku, Tokyo, Japan 107

Sony Computer Entertainment America Inc.

919 E. Hillsdale Blvd., 2nd Floor

Foster City, CA 94404, USA

Sony Computer Entertainment Europe

Waverley House

7-12 Noel Street

London W1V 4HH, England

3

Table of Contents

1. INTRODUCTION 6

a) What Can the Performance Analyzer Do? 6

2. FLOW OF DIAGNOSIS 6

(a) Measurement 6

(b) Determining whether the bottleneck is in the CPU or GPU 7

b) When a CPU process is to be tuned 7

(a) Collecting a total statistical amount to determine whether a desired performance

improvement can be achieved by tuning 7

(b) Finding the cause of an instruction cache miss 7

(c) Detecting duplicate reads from main RAM 7

(d) Detecting a write buffer flush penalty 7

c) When a GPU process is to be tuned 8

(a) Checking for null packets 8

(b) Checking the drawing start point 8

(c) Checking areas with a low drawing efficiency 8

(d) Detecting a texture cache miss 8

(e) Detecting CLUT switching 9

(f) Checking whether the resolution of the ordering table is too high 9

(g) Detecting transparent pixels 9

(h) Detecting a GPU preprocessing bottleneck 9

(I) Detecting polygons with no drawing area, back-face polygons, and polygons

subject to GPU clipping 9

(j) Checking the time required to draw a background 10

3. MEASUREMENT TECHNIQUES 10

4. INTERPRETING MEASURED DATA 12

a) Reading Analysis 14

(1) Start of a CPU process 14

(2) End of a CPU process (detection of VSync wait state) 14

(3) Start of a GPU process (drawing) 14

(4) End of a GPU process (drawing) 14

(5) Clearing of the ordering table 14

(6) Instruction cache miss 14

(7) On-cache pattern 14

(8) Interrupt 15

(9) GPU packet read 15

(10) Null packet 15

(11) Ordering table resolution 15

(12) Background drawing (texture mapping) 15

(13) Pattern exhibiting low drawing efficiency 16

(14) Pattern exhibiting a high drawing efficiency 16

(15) Semi-transparent polygon 16

Performance Analyzer Technical Manual

4

(16) When a GPU process is longer than a CPU process 17

(17) DMA transfer from the CD to main RAM 18

(18) CD read 18

(19) Transfer from main RAM to MDEC 19

(20) Transfer from MDEC to main RAM 19

b) MEASUREMENT OF A TOTAL STATISTICAL AMOUNT 20

c) DETAILS OF ANALYSIS 21

(a) Detection of the Instruction Cache Miss Function 21

(b) Detection of Duplicate Data Reads 25

(c) Detection of a Write Buffer Flush Penalty 27

(d) Detection of Null Packets 28

(e) Detection of Inefficient Texture Cell Reads 29

(f) Detection of Transparent Colors 37

(g) CLUT Switching 39

(h) Preprocessing Bottleneck 39

(I) Polygon Penalties 40

Performance Analyzer Technical Manual

5

List of Figures

Figure 1 Motortoon Grand Prix 2 (scene 1).................................................................................. 12

Figure 2 Motortoon Grand Prix 2 (scene 2).................................................................................. 13

Figure 3 V-blank interrupt ........................................................................................................... 14

Figure 4 Background section specification .................................................................................. 15

Figure 5 Background drawing ................................................................................................... .16

Figure 6 When a GPU process is longer than a CPU process....................................................... 17

Figure 7 Streaming ..................................................................................................................... 18

Figure 8 Streaming (enlarged) ..................................................................................................... 19

Figure 9 Statistical information (CPU process) ............................................................................. 20

Figure 10 Statistical information (GPU process) ........................................................................... 21

Figure 11 zimen\tuto5.cpe..................................................................................................... ..... 22

Figure 12 zimen\tuto5.cpe (instruction cache miss portion enlarged)............................................ 22

Figure 13 Function that encountered an instruction cache miss (1).............................................. 23

Figure 14 Function that encountered an instruction cache miss (2).............................................. 24

Figure 15 Statistical information of a portion where an instruction cache miss occurred ................ 25

Figure 16 Duplicate read of data from main RAM (read/write penalty).......................................... 25

Figure 17 Duplicate read of data from main RAM (data dump 1) ................................................. 26

Figure 18 Duplicate data read from main RAM (data dump 2) ..................................................... 26

Figure 19 Enlargement of a portion including a write buffer flush penalty ..................................... 27

Figure 20 Write buffer flush penalty (data dump) ......................................................................... 28

Figure 21 Null packet detection................................................................................................... 29

Figure 22 Drawing efficiency check ............................................................................................. 30

Figure 23 Portion containing more texture cache misses.............................................................. 31

Figure 24 Portion containing fewer texture cache misses ............................................................. 32

Figure 25 Portion of a high drawing efficiency (video RAM bus analysis)..................................... 33

Figure 26 Portion having a high drawing efficiency (video RAM viewer) ....................................... 33

Figure 27 Portion having a low drawing efficiency (video RAM bus analysis)............................... 34

Figure 28 Portion having a low drawing efficiency (video RAM viewer)......................................... 34

Figure 29 Sample program (without mip-mapping) ...................................................................... 35

Figure 30 Sample program (with mip-mapping) ........................................................................... 36

Figure 31 A polygon including transparent colors........................................................................ 37

Figure 32 A polygon including transparent colors (video RAM viewer).......................................... 38

Figure 33 CLUT switching and polygons requiring considerable preprocessing............................. 39

Figure 34 Polygon penalties ................................................................................................... .... 40

Performance Analyzer Technical Manual

6

1. INTRODUCTION

The performance analyzer visualizes information such as bus traffic by sampling the signals in the “PlayStation”. Some

expertise is required to tune a program based on such information. This manual details the expertise required. This manual

assumes that the user has read the "Performance Analyzer User's Manual" and is familiar with the method of using the

performance analyzer. This manual also assumes that the user is familiar with the architecture and programming of the

“PlayStation”, including programming techniques specific to high-speed processing in the “PlayStation”.

a) What Can the Performance Analyzer Do?

The performance analyzer enables its user to obtain information processed by the CPU and GPU. Based on this

information, the user can use programs to determine, for example, where the CPU is stalling, or the cause of reduced

drawing performance. However, the information obtained from the performance analyzer is, in itself, not enough to solve all

the problems that may occur. For example, the performance analyzer can detect cache misses, duplicate read accesses

and similar problems, but cannot indicate whether the actual algorithm of the program is satisfactory. Some programs may

be capable of much faster processing if the problem-solving techniques that they apply, including their algorithms, are

modified. For example, suppose that the user always wants to process all the circuit data of a racing game. This would

result in there being too much global data for the CPU to process for the polygons to be displayed. Tuning alone cannot

overcome this CPU bottleneck. To overcome this problem, high-speed processing should be implemented by the

application of a problem solving technique -- an example would be to divide the global data into groups to enable efficient

data access. Unfortunately, the performance analyzer cannot automatically analyze problem solving techniques to identify

such problems. Therefore, the programmer is responsible for determining whether a tuning technique is appropriate. The

performance analyzer should be used as a measuring tool to help the programmer make this decision.

The performance analyzer samples hardware signals directly, allowing it to double as a debugger. Since the performance

analyzer cannot measure signals in the instruction cache, scratch pad, and other devices inside the chip, however, its use is

limited as an alternative to a full-featured debugger or in-circuit emulator.

In summary, the performance analyzer can be used to:

· Determine the degree to which the current algorithm can improve performance, and identify those locations

where problems have arisen.

· Determine the currently available processing margin and how many more polygons could be displayed with

that margin, if any.

· Measure the processing speed of each candidate technique while considering which algorithm to use, and

also in the graphic design phase.

· Measure undesirable phenomena caused in real time by using the trigger function.

2. FLOW OF DIAGNOSIS

The “PlayStation” includes devices such as a CPU, GTE, GPU, MDEC, and DMA controller. All operate independently of

each other. The “PlayStation” can be thought of as a system in which a CPU and GPU operate in parallel. In many cases,

therefore, programs that run on the “PlayStation” are developed using the double-buffer method so that a CPU process

and GPU process can run concurrently to enable higher-speed processing. This means that the tuning of the CPU must be

balanced with that of the GPU to achieve high-speed processing. By monitoring the CPU and GPU buses, the performance

analyzer visualizes the processing states of the devices through main RAM bus analysis and video memory bus analysis.

Based on the above information, a program is diagnosed using the performance analyzer. The flow of the diagnosis is

outlined below.

(a) Measurement

Perform measurement of degraded processing, particularly of an overloaded scene or scene to be improved.

Detect degraded scenes using the trigger function.

Performance Analyzer Technical Manual

7

(b) Determining whether the bottleneck is in the CPU or GPU

Upon the completion of measurement, the results are displayed. From the main RAM bus analysis and video

RAM bus analysis, find the processing end points of a CPU process and GPU process. From the processing

end points, determine the process to be tuned.

b) When a CPU process is to be tuned

(a) Collecting a total statistical amount to determine whether a desired performance improvement can

be achieved by tuning

Align markers M1 and M2 with the start and end points of a CPU process, respectively, to collect statistical

information. From an estimate of the number of CPU stall cycles, check whether the desired improvement can be

made using the available tuning methods. If the check reveals that an improvement can be made, perform tuning

as described below.

(b) Finding the cause of an instruction cache miss

If an instruction cache miss produces a long CPU stall time, make improvements mainly where a stable pattern

endures for a long time. A portion with a stable pattern is regarded as performing loop processing, and further

improvement can be expected with less work. (This concept applies to improvements in data read/write access,

detailed later.) Enlarge such a section, read the symbol information, then detect the global symbols accessed in

that section to identify a function that causes a conflict in the instruction cache. Then, perform improvement by

changing the address of such a function, or by using inline expansion or DMPSX. Note, however, that the

method of changing the address of a function should be employed in the last stage because the method is

affected by modifications to the program source code.

(c) Detecting duplicate reads from main RAM

The “PlayStation” has no data cache. So, when a read access is made to main RAM as a result of a load

instruction, the CPU stalls for four cycles. This means that duplicate memory access should be avoided. Instead,

the scratch pad or a register should be used whenever possible. The performance analyzer displays the

read/write penalty "duplicate read" for a portion in which multiple read accesses have occurred successively with

no write operation performed. The ratio of the area of such a portion is regarded as representing a CPU stall time

caused by duplicate reads. For higher-speed processing, emphasis should be placed on finding such a section.

Some causes are listed below, together with the corresponding countermeasures.

(a) Move global data to the scratch pad before processing the global data.

(b) If a stack area is accessed, pass the argument(s) of a function to a register after reducing the nesting

level and the number of arguments. Or, allocate a stack in the scratch pad. (In the latter case, however,

note that the program may crash if the scratch pad overflows.)

(c) If a long expression is coded, some compilers may allocate a temporary variable in a stack area. This is

because such compilers assume no penalty in stack area access. In this case, decompose a long

expression explicitly, and use a register variable instead of a temporary variable. Or, allocate a temporary

variable in the scratch pad.

(d) Multiple half-word or byte-data accesses to adjacent areas eventually become multiple long-word

accesses. So, arrange the processing such that memory is accessed once on a long-word basis, with

conversion from long word to half-word or byte data being made between register variables.

(d) Detecting a write buffer flush penalty

With the “PlayStation”, a data write from the CPU to main RAM is performed through a four-stage write buffer.

This means that, usually, no penalty is incurred until the write buffer is full. If a write access is immediately followed

by a read access, however, the main RAM bus is not released until the write buffer has been entirely flushed. In

this case, the CPU stalls for about four cycles. The performance analyzer indicates such a stall time by the write

buffer penalty "flush penalty." Perform analysis, focusing on those portions that incur a long stall time, then

improve the efficiency by, for example, changing the order of reads and writes, and moving an instruction, if

possible, to a point after a write access.

If a CPU write cycle is immediately followed by another access on the main RAM bus, this analysis may indicate a

four-cycle penalty even when the CPU is actually executing another instruction. Thus, a penalty may be indicated

Performance Analyzer Technical Manual

8

when no stall has occurred. Therefore, assume that penalty indications merely represent the possibility of the

CPU having stalled due to flushing.

A flush penalty occurs, for example, when store and load instructions for the main RAM spaces are executed

alternately. In this case, the number of stall cycles can be reduced significantly by performing four write accesses

at a time.

c) When a GPU process is to be tuned

(a) Checking for null packets

GPU packet analysis performs a check for null packets. If successive null packets are detected, the CPU tends

to stall because drawing stops and the main RAM bus is occupied accordingly. This problem often occurs when

a background packet is placed at the start of the ordering table, or when a space is drawn which has no object

at a given depth. If idle times caused by null packets cannot be ignored, place the background packet next to a

polygon placed at the far end to start drawing there, or use multiple ordering tables.

(b) Checking the drawing start point

Drawing is started by calling a function such as GsDrawOt. Check whether time-consuming processing is

inserted between the establishment of V blank synchronization (with a function such as VSync) and the start of

drawing.

(c) Checking areas with a low drawing efficiency

A video RAM bus analysis indicates the amount of data being transferred over the video RAM bus. A higher

value represents a higher transfer rate. A height approaching 100% is indicated for a transfer rate of 32 bits per

clock cycle. On the other hand, a half height represents a maximum rate, for example, in the drawing of a

polygon. Actually, however, the write transfer rate (indicated in green) is reduced for causes such as texture

reading, as described later. Check for an extremely low green pattern. Enlarge such a pattern, if any, to

determine the cause.

(d) Detecting a texture cache miss

In texture mapping, the GPU moves texture data to the internal texture cache before starting drawing. The size of

the texture cache is limited, however, so that a cache miss may occur frequently for polygons with a large texture

or for far polygons with an excessively high texture resolution. If a cache miss occurs, the GPU stops writing to

video RAM, and loads texture data into the texture cache. If video RAM read/write cycle switching occurs

frequently, an extremely low transfer rate results. If a decrease in the drawing efficiency is caused by texture

cache misses, enclose the polygon with markers M1 and M2, then check the write area and read area with the

video RAM viewer to determine the size and direction of the drawn polygon, as well as the access roughness,

direction, and color representing an access frequency of the texture area. Then, perform cause analysis as

described below.

(a) For one polygon, the texture area accessed for reads is extremely large.

This means that the texture data is too large to be held in the cache. When texture data is to be shared

by multiple polygons, the processing can be speeded up if the texture data can be held on the cache. As

described later, a decrease in processing speed, depending on the direction of rotation, of a polygon

can be avoided by reducing the size of the texture data, even when the data is used only once. Reduce

the sizes of the polygon and texture data, or use 4-bit texture data.

(b) For a small write area, a large texture read area is used.

This means that the texture resolution is too high. With the video RAM viewer, a texture area access

pattern is represented by thin horizontal parallel lines. Namely, when a texture cache miss occurs, 64-bit

texture cells arranged horizontally and extending to the right from the texture area are read, regardless of

the texture resolution. This read operation results in a thin horizontal line. Vertically, however, texture cells

are not read continuously, but are skipped. Thus, an access pattern like that described above results.

For correction, reduce the texture resolution. If the resolution varies over a wide range, use mip-mapping

or mip-modeling.

(c) The texture access area is long vertically. The texture read frequency is greater than the drawing

frequency.

If a texture cache miss occurs, texture cells are read horizontally as explained above. When a vertical bar

Performance Analyzer Technical Manual

9

is drawn, for example, not all of the read texture cells may be used. In such a case, a higher efficiency

may be achieved by placing the texture data horizontally.

With a rotating polygon, the texture read efficiency changes. A poor efficiency results particularly when

the texture read direction differs from the drawing direction by 90 degrees. In this case, when the

drawing section of a polygon is viewed using the video RAM viewer, the frequency of reading from the

texture area tends to be displayed in a color that represents a higher frequency than that of writing to the

drawing area. Several countermeasures can be applied. Divide the polygon so that the polygon data can

be held in the cache. Read from the texture area by rotating the texture pattern so as to ensure efficient

read access at all times. Or, provide multiple texture patterns to match the directions of rotation and to

switch between them.

(e) Detecting CLUT switching

Compared with an 8-bit texture, a 4-bit texture allows double the number of texture cells to be held in the texture

cache. However, a 4-bit texture supports only a limited number of colors, so frequent CLUT switching will often

be required. The “PlayStation” uses the Z sort method for drawing, such that CLUT control cannot be applied

explicitly. If CLUT switching occurs frequently with the same Z value, the time required for CLUT switching may

become so large that it can not be ignored. With the performance analyzer, CLUT reads, when displayed, are

colored by the video RAM bus analysis, enabling CLUT switching to be identified. If the texture resolution is high,

and texture cells are used on a skipping basis, many texture cells that are not used may be read even when an

attempt is made to improve the cache efficiency by using a 4-bit texture. If this occurs, the use of a 4-bit texture

does not speed up processing, compared with that possible with an 8-bit texture. If the penalty incurred by CLUT

switching cannot be ignored in such a case, the use of an 8-bit texture with an increased number of colors can

eliminate CLUT switching, thus speeding up drawing.

(f) Checking whether the resolution of the ordering table is too high

If the resolution of the ordering table is too high, an excessive number of null packets will be read, such that the

drawing efficiency decreases. Check this point carefully.

(g) Detecting transparent pixels

When a tree is to be drawn, for example, this can be done by creating a texture with a transparent color and

representing the tree with one quadrangle polygon. This method is not efficient, however, because the drawing

of a transparent color, including the reading of a transparent texture and the drawing of pixels, requires as much

time as the drawing of ordinary pixels. So, divide the object in such a way that the leaves are represented by

triangle polygons, and the trunk by thin rectangles, such that the transparent area is minimized.

(h) Detecting a GPU preprocessing bottleneck

GPU processing can be divided into two steps: preprocessing for converting packet data to parameters

required for the drawing engine internal to the GPU, and drawing processing. The time required for drawing

processing is roughly proportional to the drawing area. However, the preprocessing time depends not on the

drawing area, but on the type of the polygon. So, for a small polygon placed at the far end, most of the GPU

processing time is generally used for preprocessing. Thus, preprocessing tends to reduce the drawing

efficiency. In particular, when there are many small polygons with a Gouraud texture that require much

preprocessing, the drawing efficiency drops considerably. Using the performance analyzer, such a

preprocessing bottleneck can be detected by applying GPU packet analysis. A portion in which packets

requiring much preprocessing have high patterns involves many small polygons that tend to cause a

preprocessing bottleneck. If polygons that have small drawing areas and which require much preprocessing

need not be used, replace such polygons with other polygons that require relatively little preprocessing when

creating packets.

(i) Detecting polygons with no drawing area, back-face polygons, and polygons subject to GPU

clipping

A polygon that has no area requires longer preprocessing time than ordinary polygons do. So, before packets

are registered in the ordering table, the areas of the constituent polygons should be checked; this check also

serves to prevent useless polygons from being sent to the GPU.

Performance Analyzer Technical Manual

10

Check also whether there are any back-face polygons for which normal clipping is not performed.

The GPU can perform two-dimensional clipping by hardware. However, a polygon that reaches the left and

upper boundaries of the screen requires the same amount of processing time as when the polygon is not

clipped. If there are many such polygons, the drawing efficiency decreases. The performance analyzer displays

the distribution of these polygons by using polygon penalties.

To check for polygons with no area, an outer product value returned from the GTE function can be used. Note,

however, that when a quadrangle polygon is checked, the area of one of the two partial triangles is returned, and

a gap can result.

(j) Checking the time required to draw a background

Usually, a background is drawn by using sprites. However, a longer drawing time may be consumed by

duplicate drawing or drawing using polygons. Using video RAM bus analysis, check whether the time required to

draw a background is sufficiently short.

The typical flow of tuning using the performance analyzer is described above. This method may not be able to solve many

other problems, which may be associated with the use of libraries, or may be specific to individual programs. In such a

case, contact the development support section.

3. MEASUREMENT TECHNIQUES

With the performance analyzer, a V-blank interrupt serves as a reference point for measurement data. When a program with

one frame containing 2 V (two V-blank interrupts) or more is to be measured, the first reference point does not always

represent the start point of the main loop. To store data by matching a reference point with the start point of frame

processing, use one of the methods described below.

(a) Using synchronized data after repeated measurements

This is the simplest method. When a longer measurement time is to be set because of a low frame rate

that is caused, for example, by degraded processing, the probability of start point synchronization is

reduced. Accordingly, the measurement will have to be repeated. After measuring the data, stop the data

transfer before it ends. Then, display the results of main RAM bus and video RAM bus analysis for

100,000 cycles. At the start of a loop, a pattern for clearing the ordering table occurs on the main RAM

bus, and a pattern for clearing the background occurs on the video RAM bus. Continue to take these

measurements until these patterns are obtained. When these patterns are obtained, do not take a

measurement, but instead transfer data to obtain the entire data.

(b) Sampling for a longer period of time, then clipping and saving the required portion

Measure the section to be measured by doubling the number of V blanks, then clip and save the required

portion. With a program that has a low frame rate, however, a measurement section may be too large to

be held in the memory of the performance analyzer. Moreover, if measurement is repeated several times,

the data transfer time is doubled, thus resulting in reduced efficiency.

(c) Using the trigger function

The trigger function of the performance analyzer can be used. For example, GsDrawOt (executed after

VSync in the main loop) is used as a trigger address. After reading the symbol information, check the

main RAM access address item in the trigger setting dialog box, then select a function address to be

used as a trigger condition. An instruction cache miss must occur to enable the performance analyzer to

detect a selected address. When addresses are accessed in the main loop, however, one cache miss is

expected to occur for each frame, thus enabling synchronization to be established.

The above methods may not be suitable for measuring a phenomenon that occurs only rarely. For example, if the frame

rate drops in a rare case, the pinpoint measurement of such a phenomenon should be performed. For this purpose, use a

counter, for example, to find the time required to process the frame before calling VSync. Then, set up the processing such

that a particular global symbol in main RAM is accessed when the frame rate drops. Namely, modify the program as

follows:

Performance Analyzer Technical Manual

11

#define HCountThreshold 525 /* frame length in H count (needs an adjustment) */

volatile long ReqTrigger;

main()

{

...

for(;;){ /* main loop */

...

/* compare H-counts since the last VSync exited */

if(VSync(1) > HCountThreshold)

ReqTrigger = 0;

VSync(0);

...

}

}

Thus, frame rate drop can be measured by setting an access to the ReqTrigger variable as a trigger condition, and setting

NFV of the trigger condition to the number of V blanks applied to one frame.

By modifying a program as described above, a variety of phenomena can be measured using the trigger function.

Performance Analyzer Technical Manual

12

4. INTERPRETING MEASURED DATA

The following describes how to interpret the measured data.

Figure 1 Motortoon Grand Prix 2 (scene 1)

Performance Analyzer Technical Manual

13

Figure 2 Motortoon Grand Prix 2 (scene 2)

Figures 1 and 2 show the results of measuring the car racing game, Motortoon Grand Prix 2. This program employs the

double-buffer method, a standard “PlayStation” programming technique. Analysis indicate that the main RAM bus and

video RAM bus are accessed at the same time so that a CPU process and GPU process are performed concurrently. In

Figures 1 and 2, a 2V section is measured; the V blank positions are indicated by red lines on the upper ruler. (Figures 1 and

2 are based on NTSC. For PAL, these red lines are spaced further apart.) How to read the patterns numbered in Figures 1

and 2 is described below.

Performance Analyzer Technical Manual

14

a) Reading Analysis

( 1) Start of a CPU process

Figure 3 V-blank interrupt

A CPU process starts when the VSync function ends in the program. The measurement data first indicates an

interrupt routine caused by a V-blank interrupt. In Figure 3, the starting portion is enlarged. As can be seen from

Figure 3, about 20,000 clock cycles are required for interrupt routine processing, after which the user program is

executed. In Figure 1, this point acts as the start point of a CPU process. (This means that symbol main is the first

point to be accessed.)

( 2) End of a CPU process (detection of VSync wait state)

The end of a CPU process is not detected automatically. However, the end point can be obtained by finding a

stable read/write pattern that appears at the end of the processing. This is enabled by the VSync function polling

the variables set by the interrupt routine. Such a stable pattern often represents loop processing. The end of a

CPU process corresponds to the start of a pattern of the VSync function. Usually, the VSync function operates

on the cache, so that no red pattern, representing a cache miss, appears.

( 3) Start of a GPU process (drawing)

The start of drawing is represented by the first access revealed by video RAM bus analysis.

( 4) End of a GPU process (drawing)

The end of drawing is represented by the last access revealed by video RAM bus analysis.

( 5) Clearing of the ordering table

The ordering table is cleared using a function such as ClearOT. The DMA controller in the CPU chip writes a long

word to main RAM in each clock cycle. Thus, a high peak appears in the first half of the processing.

( 6) Instruction cache miss

A burst read from memory, caused by an instruction cache miss, is indicated by a red pattern.

( 7) On-cache pattern

If the program causes no instruction cache miss, no red pattern is displayed, as indicated here. In this case, the

CPU operates efficiently without stalling.

Performance Analyzer Technical Manual

15

( 8) Interrupt

Usually, an interrupt is a V-blank interrupt. Other types of interrupts, such as sound interrupts and timer interrupts,

can be generated. When an interrupt is generated, an instruction cache miss occurs and a red spike-like pattern

is produced, as indicated here. If such an interrupt is generated frequently, the main task processing is impeded.

So, be careful if such a pattern is detected frequently.

( 9) GPU packet read

The GPU usually reads packets from main RAM by means of DMA transfer. This pattern is represented in pink.

When more data is transferred, a thicker pattern results. Thus, main RAM bus access by the CPU is impeded,

causing the CPU to stall.

(10) Null packet

A null packet is a packet for which there is no entry on the ordering table. If null packets occur in succession, no

drawing packet is transferred to the GPU, such that drawing is delayed accordingly. GPU packet analysis uses a

white pattern to represent null packets. Null packets appearing in succession not only delay drawing by the GPU,

but also cause frequent useless GPU packet reads, as can be seen from a main RAM bus analysis; thus, the

availability of the main RAM bus becomes very low.

(11) Ordering table resolution

A white pattern mixed with polygon packets like this indicates reduced drawing efficiency, caused by the ordering

table resolution being too high. By means of video RAM bus analysis, enlarge and check this portion to

determine the adverse effect of high resolution on the processing.

(12) Background drawing (texture mapping)

Figure 4 Background section specification

Performance Analyzer Technical Manual

16

Figure 5 Background drawing

To determine a background boundary, enclose the first pattern in the video RAM bus analysis with the M1 and

M2 markers as shown in Figure 4. Then, obtain the drawing area by executing the video RAM viewer command,

as shown in Figure 5, and check that a background is drawn. An access frequency is represented by a color,

enabling a doubly drawn area to be detected. If the resolution of the texture is too high in background drawing,

video RAM bus analysis indicates texture reads in red and a lower write pattern in green. If background drawing

takes a long time, tuning should take this point into consideration.

(13) Pattern exhibiting low drawing efficiency

A low green pattern like that in this portion represents a low drawing transfer rate. For troubleshooting, enlarge

such a portion.

(14) Pattern exhibiting a high drawing efficiency

Efficient drawing is indicated by a pattern like this. No texture reads or CLUT reads are performed. Moreover, the

time required for polygon preprocessing can be ignored, relative to the drawing time, and a high green write

pattern is indicated. In polygon drawing, the pattern height can increase by up to 50%.

(15) Semi-transparent polygon

A semi-transparent polygon is represented by a navy blue pattern. Note that the drawing of a semi-transparent

polygon takes three times longer than an ordinary write.

Performance Analyzer Technical Manual

17

(16) When a GPU process is longer than a CPU process

Figure 6 When a GPU process is longer than a CPU process

Figure 6 shows a GPU process that ends after a CPU process ends. As shown in Figure 6, even when the CPU

is performing loop processing in the VSync function, GPU packets are read on the main RAM bus. So, the

VSync wait pattern does not appear in a stable manner when compared with Figure 1. In this case, however, the

CPU has ended its processing. Moreover, it can be observed that an interrupt for drawing termination was

generated upon the completion of drawing, and a spike pattern representing an instruction cache miss due to

interrupt handling occurred on the main RAM bus.

Next, an example of measuring a streaming program is shown in Figure 7. Each numbered pattern is explained below.

Performance Analyzer Technical Manual

18

Figure 7 Streaming

(17) DMA transfer from the CD to main RAM

A pattern like this occurs when data is transferred from the CD to main RAM. A high pattern is displayed.

However, main RAM bus analysis indicates the period during which the main RAM bus is occupied. This means

that data is not always transferred in each cycle. Data transfer from the CD occupies the main RAM bus for a

long time although the amount of data is not large. If the CPU attempts to access the main RAM bus while data is

being transferred from the CD, the CPU will stall for a long time.

(18) CD read

The CD device is connected to the sub-bus, another bus used by peripheral devices. So, CD reads are

displayed by a sub-bus analysis like this.

Figure 8 shows an enlargement of part of the streaming. The patterns described below are observed.

Performance Analyzer Technical Manual

19

Figure 8 Streaming (enlarged)

(19) Transfer from main RAM to MDEC

Data transfer from main RAM to MDEC is represented by an orange pattern. Usually, compressed data is

transferred.

(20) Transfer from MDEC to main RAM

Transfer from MDEC to main RAM is regarded as an internal DMA write transfer, so that a dark green pattern

appears in the same way as when clearing the ordering table. MDEC expands data, such that data transferred

from MDEC to main RAM is decompressed image data.

Performance Analyzer Technical Manual

20

b) MEASUREMENT OF A TOTAL STATISTICAL AMOUNT

Figure 9 Statistical information (CPU process)

Statistical information can be obtained by placing markers M1 and M2 at the start and end points of a CPU process,

respectively. Figure 9 shows the statistical information thus obtained, which includes the estimated number of CPU stall

cycles for the program. By referring to that value, determine the type of tuning required. If the desired processing speed

cannot be obtained even after the total number of CPU stall cycles is reduced, another solution should be found by

determining which processing is overloaded.

Next, place markers M1 and M2 at the start and end points of a GPU process, respectively, then measure the statistical

amounts in the same way (Figure 10). Information displayed here shows the number of polygons and sprites which the GPU

processed. Such information can also be used to check whether the number of polygons to be displayed is adequate.

Performance Analyzer Technical Manual

21

Figure 10 Statistical information (GPU process)

c) DETAILS OF ANALYSIS

The methods of analysis are detailed below.

(a) Detection of the Instruction Cache Miss Function

Performance Analyzer Technical Manual

22

Figure 11 zimen\tuto5.cpe

Figure 11 shows the results of measuring the sample program, psx\sample\graphics\zimen\tuto5.cpe, provided

on the library CD-ROM. A 1V section measurement is made, and a main RAM bus analysis indicates a cache

miss, in red, in the first half. We can determine the function that caused this cache miss. First, position the cursor

to the area where the cache miss occurred, and enlarge that area. Then, the patterns shown in Figure 12 are

obtained.

Figure 12 zimen\tuto5.cpe (instruction cache miss portion enlarged)

These patterns appear cyclically. From the clock cycle values indicated on the ruler, the period is found to be

about 700 cycles. As this period is shorter, and these patterns represent a greater proportion of the overall

Performance Analyzer Technical Manual

23

processing, a greater improvement can be expected despite the application of less tuning. Here, enclose several

cyclic patterns, regarded as representing loop processing, with markers M1 and M2. Then, read the mapping

information, and display only those global symbols that are accessed in the section by selecting filtering from the

menu. (Figure 12 already indicates these global symbols.) Next, enable global symbol access display.

Furthermore, with the option menu, reset the scale factor specification for global symbol access display to enable

display using the current scale factor.

Then, the global symbols accessed in the M1-M2 section are displayed as shown in Figure 12. Those functions

that caused instruction cache misses are displayed in red. We can determine which functions caused a

particularly large number of instruction cache misses. Figures 13 and 14 show the data dumped by positioning

the cursor to each function and double-clicking.

Figure 13 Function that encountered an instruction cache miss (1)

Performance Analyzer Technical Manual

24

Figure 14 Function that encountered an instruction cache miss (2)

The size of the “PlayStation”'s instruction cache is 4KB. The cache line size is 4 long words, and 256 cache lines

are allowed. As shown in Figure 13, the lower two digits next to the lowest digit of a main RAM bus address

represent the line number. This means that a cache miss is caused by instructions that are located at different

addresses but which have the same lower two digits. There are several methods of eliminating such a cache

miss. Use inline expansion or DMPSX, or link those functions to locate the functions at addresses close to each

other if the functions are user functions. Note, however, that a cache miss is unavoidable if the loop includes

code of 4KB or more. One speedup technique for the “PlayStation” involves ensuring that no small loop includes

a code of 4KB or more.

Next, position M1 and M2 to cover the section including all the patterns for loop processing involving a cache

miss, then collect the statistical information for the section (Figure 15).

Performance Analyzer Technical Manual

25

Figure 15 Statistical information of a portion where an instruction cache miss occurred

The CPU stall time caused by an instruction cache miss corresponds to about half of the red indication provided

by main RAM bus analysis. Here, the CPU stall time is represented by the number of clock cycles. If cache

misses can be eliminated from this section, processing can be speeded up by the corresponding number of

clock cycles.

(b) Detection of Duplicate Data Reads

A CPU data read cycle on the main RAM bus is indicated as a yellow pattern by main RAM bus analysis. The

CPU stall time corresponds to the total yellow area. Data such as global symbols may be read from main RAM,

but a work area and temporary variables should be accessed using, for example, the scratch pad. If main RAM

must still be accessed, tuning can be achieved by checking whether there is a duplicate read, that is, by

checking whether there is an access for reading from the same address more than once without writing. In

Figure 11, such a duplicate read is indicated by the read/write penalty "duplicate read." A red area represents a

duplicate read, or CPU stall time. Figure 16 shows an enlarged view of a portion that includes many read

patterns. Here, position the cursor to a red pattern, then double-click to dump the data (Figure 17).

Figure 16 Duplicate read of data from main RAM (read/write penalty)

Performance Analyzer Technical Manual

26

Figure 17 Duplicate read of data from main RAM (data dump 1)

Next, by using the search command, identify an address that is accessed more than once. Figure 18 shows the

dumped data.

Figure 18 Duplicate data read from main RAM (data dump 2)

The reason for such a duplicate read is described in Flow of Diagnosis. The code should be modified,

depending on the cause, to eliminate duplicate reads whenever possible.

A byte access or short-word access is handled as a long-word access on the main RAM bus. This means that,

even when a read access which is not duplicated is coded, the performance analyzer may indicate the access

as a duplicate read.

Performance Analyzer Technical Manual

27

The CPU stall time for one read access is four cycles. So, if a code for using the scratch pad or a register to

avoid access to main RAM is shorter than the number of duplicate accesses multiplied by four instructions, the

use of such code will speed up processing.

A duplicate global symbol read can be identified from global symbol access and data dumping. However,

access to a stack area requires some care. Namely, when a symbol file is read without first setting a stack area

using the performance analyzer, an access to a stack area is mistakenly indicated as a highest-level access to a

global symbol. In such a case, an access to a stack causes an extremely high offset value for a particular global

variable.

(c) Detection of a Write Buffer Flush Penalty

In Figure 11, a write buffer analysis indicates a portion where a device such as the CPU may stall because an

access request is generated while the write buffer is being flushed. The area represents the stall time. For tuning,

the portion where many red patterns are visible should be carefully checked.

Figure 19 Enlargement of a portion including a write buffer flush penalty

Figure 19 shows an enlarged view of the portion; at the cursor position, a CPU read access occurred

immediately after a write access. Figure 20 shows the data dump, displayed by double-clicking.

Performance Analyzer Technical Manual

28

Figure 20 Write buffer flush penalty (data dump)

Check the code. If a store instruction is immediately followed by a load instruction, reduce the stall time by

inserting another instruction between the two instructions or by exchanging the two instructions with each other,

if possible. Particularly, when a read and write occur alternately and repeatedly, a longer stall time results. In such

a case, take advantage of the four stages of the write buffer. That is, the stall time can be dramatically reduced

by modifying the code so that four writes occur in succession. When 100,000 polygons are to be displayed per

second, for example, the length of a loop for processing one polygon will be about 200 to 300 cycles. If a stall

time of 10 cycles is reduced by tuning the write buffer, an improvement of 3% to 5% is achieved. This means that

the number of polygons to be displayed can be increased by such an improvement. Note that, even if an

instruction other than a store instruction is executed immediately after flushing, a write buffer access causes a red

pattern when a read access occurs immediately after on the main RAM bus. Patterns do not always represent

penalties.

(d) Detection of Null Packets

As shown by the GPU packet analysis in Figure 1, a null packet is represented by a white pattern. White patterns

occurring in succession represent successive null packets in the ordering table. Successive null packets can be

eliminated using multiple ordering tables, for example.

An enlarged view of a null packet indicates that the drawing is delayed accordingly, as shown in Figure 21.

Performance Analyzer Technical Manual

29

Figure 21 Null packet detection

If null packets are mixed with other polygon packets, the drawing efficiency may have decreased because the

resolution of the ordering table is too high. If video RAM bus analysis indicates that the green pattern is low, the

problem may be solved by lowering the resolution.

(e) Detection of Inefficient Texture Cell Reads

Performance Analyzer Technical Manual

30

Figure 22 Drawing efficiency check

As shown in Figure 22, position markers M1 and M2 to a portion with fewer green patterns on the video RAM

bus, and enlarge the enclosed portion. In this case, the pattern shown in Figure 23 is obtained.

Performance Analyzer Technical Manual

31

Figure 23 Portion containing more texture cache misses

Next, position markers M1 and M2 to that portion having more green patterns, and enlarge the portion. In this

case, the pattern shown in Figure 24 is obtained.

Performance Analyzer Technical Manual

32

Figure 24 Portion containing fewer texture cache misses

The two red stripes produced by video RAM bus analysis represent a 64-bit texture read. For texture read, the

texture cells corresponding to the line size of the texture cache are read from video RAM if a texture cache miss

occurs; the texture cells read at one time are always 64 bits long.

Figure 25 shows an enlarged view of a portion for which the drawing efficiency is high. When such a portion is

checked with the video RAM viewer, a pattern like that shown in Figure 26 is obtained. On the other hand, when

a portion for which the drawing efficiency is poor is enlarged, as shown in Figure 27, and is checked with the

video RAM viewer, a pattern like that as shown in Figure 28 is obtained.

Performance Analyzer Technical Manual

33

Figure 25 Portion of a high drawing efficiency (video RAM bus analysis)

Figure 26 Portion having a high drawing efficiency (video RAM viewer)

Performance Analyzer Technical Manual

34

Figure 27 Portion having a low drawing efficiency (video RAM bus analysis)

Figure 28 Portion having a low drawing efficiency (video RAM viewer)

In a texture cache line fill, horizontally successive texture cells of the texture area are read. So, if the resolution of

the texture is too high, and 4-bit texture mode is used, for example, 15 out of 16 texture cells may be discarded,

and only one pixel may be drawn. An example is shown in Figure 23. In this example, only one pixel is drawn for

each texture read caused by a texture cache miss. When Figure 26 is compared with Figure 28, it can be seen

that more texture reads are performed in Figure 28.

A similar phenomenon occurs when a texture larger than the texture cache is used, and when a polygon to be

drawn is rotated through 90 degrees relative to the texture pattern. Each cause can be identified from read

access and write access patterns with the video RAM viewer. Apply appropriate action such as changing the

texture size and texture resolution, depending on the identified cause.

Figures 29 and 30 show the improvement realized for the sample program,

Performance Analyzer Technical Manual

35

psx\sample\graphics\mipmap\tuto5.cpe, made by mip-mapping. These figures reveal that a significant

improvement in drawing speed can be achieved by applying mip-mapping.

Figure 29 Sample program (without mip-mapping)

Performance Analyzer Technical Manual

36

Figure 30 Sample program (with mip-mapping)

Performance Analyzer Technical Manual

37

(f) Detection of Transparent Colors

Figure 31 A polygon including transparent colors

Based on GPU packet analysis, Figure 31 shows an enlarged view of a section containing no accesses. From

the GPU packet analysis, the drawing of one large polygon is assumed. Video RAM bus analysis indicates that

the texture is read constantly, but that drawing is not performed in many portions. Figure 32 shows the

information obtained with the video RAM viewer by positioning M1 and M2 such that they enclose the drawing

section of a polygon.

Performance Analyzer Technical Manual

38

Figure 32 A polygon including transparent colors (video RAM viewer)

Usually, with the video RAM viewer, the left side represents the frame buffer, while the right side represents the

texture area. When the double-buffer method is used, the frame area is divided into an upper area and lower

area; each time the frame is switched, the access area is switched. Figure 32 shows that a transparent color is

used in the frame buffer area. Thus, a polygon causing a problem can be identified using the video RAM viewer.

A polygon with large transparent area should be divided to reduce the size of transparent area.

Performance Analyzer Technical Manual

39

(g) CLUT Switching

Figure 33 CLUT switching and polygons requiring considerable preprocessing

Another example having poor drawing efficiency is shown in Figure 33. A video RAM bus analysis shows that an

orange pattern representing a 4-bit CLUT read occurs with each polygon. This means that many polygons use

different CLUTs with close Z values. If the video RAM bus is occupied because of frequent switching between

CLUTs, action is required.

(h) Bottleneck

The GPU packet analysis shown in Figure 33 indicates that a Gouraud texture polygon is drawn. A video RAM

bus analysis indicates that a relatively large portion involving no accesses precedes texture read or pixel drawing.

This means that a long time is required for Gouraud texture preprocessing. To speed up the processing, those

small polygons that require considerable preprocessing should not be drawn wherever possible.

Performance Analyzer Technical Manual

40

(i) Polygon Penalties

Figure 34 Polygon penalties

Figure 34 evaluates and indicates the penalties of zero-area polygons, polygons with a poor scissoring

efficiency, polygons clipped by the GPU, and back-face polygons subject to normal clipping. To improve the

processing, cause the CPU to perform normal clipping, area checking, polygon division, and area clipping for

these polygons, if the CPU has enough processing time left to do them.

The performance analyzer identifies back-face polygons checking the order of vertices in a polygon packet.

Some programs may assume such an order is for front-face polygons, or may assume any kind of order is for

valid polygons which should be drawn. In such cases set the parameter in options dialog box to display correct

polygon penalties.

Also the offset value and the screen size should be set in options dialog box to display correct polygon penalties.