DTL-H2500 Installation and Operation

Documentation Version 1.020

© 1996-1998 Sony Computer Entertainment

Sony Computer Entertainment
919 E. Hillsdale Blvd., 2nd Floor
Foster City, CA 94404

The DTL-H2500 Installation and Operation Manual is supplied pursuant to and subject to the terms of the Sony Computer Entertainment America PlayStation™ License and Development Tools Agreements or the Developer Agreement.

The DTL-H2500 Installation and Operation Manual is intended for distribution to and use by only Sony Computer Entertainment America licensed Developers and Publishers in accordance with the PlayStation™ License and Development Tools Agreements or the Developer Agreement.

Unauthorized reproduction, distribution, lending, rental or disclosure to any third party, in whole or in part, of this book is expressly prohibited by law and by the terms of the Sony Computer Entertainment America PlayStation™ License and Development Tools Agreements or the Developer Agreement.

Ownership of the physical property of the book is retained by and reserved to Sony Computer Entertainment America, a division of Sony Interactive Entertainment Inc. Alteration to or deletion, in whole or in part, of the book, its presentation or its contents is prohibited.

The information in the DTL-H2500 Installation and Operation Manual is subject to change without notice. The content of this book is Confidential Information of Sony Computer Entertainment America.

PlayStation and PlayStation logos are trademarks of Sony Computer Entertainment Inc. All other trademarks are property of their respective owners and/or their licensers.

Table of Contents

About this Manual

Manual Release History	v

Related Documentation	v

Technical Assistance	vi

Chapter 1: Hardware Installation

Before you Begin

Hardware Installation Procedure

Chapter 2: Software Installation

Before you Begin

Installing Software Drivers and Software Tools

Installing the H25DRV.EXE driver

Installing the H25BIOS.COM driver

Running from the CDROM

Debugging using the DTL-H2500

Miscellaneous

Chapter 3: Troubleshooting the installation

Preliminaries

Problem: PC fails to recognize board

Known problems with BIOS of PC

Diagnostic steps

Problem: Board will not reset/Can’t run programs

General known compatibility issues with PC Brands and models

Chapter 4: DTL-H2500 Questionnaire

Chapter 5: Case study

Chapter 6: Installing to an AMI BIOS Machine

Chapter 7: Commands for the Flash-ROM

About this Manual

This is the CDROM release of the Installation and Operation Manual for the DTL-H2500 Development System. Library functions and structures are detailed in the Library Reference volume of the PlayStation Developer Reference Series.

Discard the black diskette which came with your development system since it is now out of date. More up-to-date versions are on the CD-ROM that came with your system. The latest software and patches can be obtained from our Web Sites. Read the section "Technical Assistance" below for more details.

Manual Release History

Version	Released	Change

Ver. 1.0	18-Oct-96

Ver 1.004	18-Nov-96

Ver 1.006	29-Jan-97

Ver 1.007	20-Feb-97

Ver 1.008	12-Mar-97	Added info on running CD-ROMs from the DTL-H2510.

Ver 1.009	14-Mar-97	Corrected info on "If 'addr' is set to any of the addresses from 0x00000001 to 0x000FFFFF (1 M or less), the address mapping by the PCI will fail" to "If 'addr' is set to any of the addresses outside of the range from 0x00000001 to 0x000FFFFF (1 M or less), the address mapping by the PCI will fail." Based on information from K.S. at SCEI.

Ver 1.010	07-Jun-97	Removed references to BBS.

Ver 1.011	09-Jun-97	Added portion on running CD samples

Ver 1.012	01-Jul-97	Set “Micron” incompatible from “incompatible (?)” to definitely incompatible.

Ver 1.013	28-Jul-97	Changed the method for determining the available IO port addresses and  interrupts (you can use the "Print…" to print out the summary).

Ver 1.014	01-Aug-97	Changed "415" area code to "650". Revamped the "Technical assistance" section.

Ver 1.015	15-Aug-97	Incorporated new list of computer-compatibility issues, and merged the older table into the new table. Added the bitmap of the memory range dialog box.

Ver 1.016	18-Aug-97	Fixed typographical errors, alphabetization problems, etc., with the computer-compatibility table. Revamped the running of CD-ROMs on the DTL-H2510.

Ver 1.017	11-Sep-97	Manual was reformatted.

Ver 1.018	12-Nov-97	Changed “\psx\bin\” to “\psx\bin\DTLH2500\H25DRV” and “\pssn\bin” to “\pssn\bin\DTLh2500\H25bios” where appropriate. Added a little bit more to the dbugpsx section.

Ver 1.019	26-Nov-97	Fixed typographical errors and table headings.

Ver 1.020	15-Jul-98

Related Documentation

In addition to this document, the installation sheets which were provided with the hardware contain helpful information.

The Technical Reference CD contains documentation in "*.pdf" format, and can be read using the Adobe Acrobat readers supplied on the CDROM. Insert the Technical Reference CD into your PC and run the setup programs to install the Adobe Acrobat reader.

Note that the Web Sites post late-breaking developments regarding the Libraries and also provide notices of forthcoming documentation releases and upgrades. Read the section "Technical Assistance" below for more details.

Technical Assistance

If you are experiencing problems, we highly recommend that you first search the Technical Reference CD since this is the same tool our Technical Support group uses to attack a problem. It is highly probable that other developers have encountered the same problem in the past and that it was solved and then documented on the Technical Reference CD. However, don't bang your head against the wall! If (and only if) you are a licensed developer, you can reach technical support for your region at the following addresses and telephone numbers. For more information and for a set of bug report forms, refer to the Technical Reference CD.

SN Systems

SN Systems writes the compilers for the standard PlayStation development kits. All licensees of SCEA and SCEE are welcome to email bug reports or ask questions about the compilers.

E-mail: support@snsys.com.

Web Site: http://www.snsys.com.

Sony Computer Entertainment America

SCEA is available to licensees in North America only.

E-mail: DevTech_Support@interactive.sony.com

Web Site: http://www.scea.sony.com/dev

Developer Support Hotline: 650-655-8181, Monday through Friday, 8am to 5pm, Pacific Standard Time.

Mail: Sony Computer Entertainment America Inc., 919 East Hillsdale Blvd., 2nd Floor, Foster City CA 94404

Sony Computer Entertainment Europe

SCEE is available to licensees in Europe only.

E-mail: dev_support@interactive.sony.com

Web Site: https://www-s.playstation.co.uk

Developer Support Hotline:+44 (0) 171 390 1680

Mail: Waverley House 7-12 Noel Street London W1V 4H

# Hardware Installation

## Before you Begin

This chapter describes the installation procedures for the following equipment:

Required:

DTL-H2500 PCI card and an available PCI slot

DTL-H2500 connector card

Optional:

DTL-H2510 CD-ROM drive

CD-ROM Emulator ISA card, an available ISA slot, and an AV SCSI hard drive from the following list:

DCAS-34330	ULTRA SCSI 4.3GB/IBM

Fireball ST-3200S	ULTRA SCSI 3.2GB/Quantum

Fireball TM-3200S	ULTRA SCSI 3.2GB/Quantum

Atlas XP-4550S	ULTRA SCSI 4.5GB/Quantum

Medalist ST52160N	ULTRA SCSI 2GB/Seagate

Barracuda ST32171N	ULTRA SCSI 2GB/Seagate

## Hardware Installation Procedure

Step 1:	Turn off your computer. Discard or recycle the black diskette supplied with the DTL-H2500 kit since it is not needed.

Step 2:	Connect the DTL-H2500 connector card onto an open slot on the PC.

Step 3:	Insert the DTL-H2500 (PCI card) into an available PCI slot. 
Because the PCI card is Plug & Play, setting the IRQ’s or DMA numbers is not necessary.

Step 4:	Connect the PCI to the controller connector card.
Connect the DTL-H2500 PCI card to the DTL-H2500 connector card using the 10 pin flat cable included in your kit.

Step 5:	Connect the audio/visual outputs to your television.
Refer to the “DTL-H2500 PlayStation Board Operating Instructions” sheet for further details.

Step 6:	Attach the PlayStation controllers to the outputs of the DTL-H2500 board.
Refer to the “DTL-H2500 PlayStation Board Operating Instructions” sheet for further details.

United States developers: In the US, the DTL-K2500 Programmers Tool Kit includes the DTL-H2080 Controller Box along with 2 SCPH-1010 Controllers. If you did not receive your controllers, call the Tool Operations Group at (US) 650-655-8145.

European developers: In Europe, the DTL-H2500 PlayStation development board is licensed as a separate unit and does not include controllers. We recommend you order a DTL-H2080 Controller Box with your DTL-H2500; call the Tool Operations Group at (Europe) 0171-4471650.

Step 7:	(Optional) Install the DTL-H2510 CD-ROM drive
The CD-ROM drive reads PlayStation debugging disks. Follow these steps:

- Place the drive in an open drive bay on the PC.

- Connect the 50 pin flat cable, included in your kit, to the CD-ROM drive.

- Connect the other end of the 50 pin flat cable of the CD-ROM drive to the DTL-H2500 board.

- Connect the 4 pin hard-drive power cable into the CD-ROM drive's power inlet.

Step 8:	(Optional) Install the emulator card.
If you have a CD-ROM emulator card (PSX-04 or  DTL-K4) and a dedicated AV SCSI hard drive (for more details, refer to the manual “DTL-H2510 CD-ROM drive”), please read the comprehensive "readme.txt" that came with the software in that kit. The "readme.txt" explains how to install the card. However, you should defer this installation until after you are certain that your PCI card is functioning properly.

Please make a note of the address settings, DMA, and IRQ on the ISA card:

DMA:			__________

IRQ:			__________

IO Address:		__________

Although the emulator card's actual address is in 4 byte hexadecimals, the DIP switch host’s A15-A4 3 bytes are in decimal format. The actual addresses and a table of their equivalents are entered below:

IO Address	Hex	Actual 	Remarks

Decimal Notation	Notation	Address

300	0x12C	0x12C0

308	0x134	0x1340	Default

310	0x136	0x1360

318	0x13E	0x13E0

380	0x17C	0x17C0

388	0x184	0x1840

390	0x186	0x1860

398	0x18E	0x18E0

In this case, take A15-A4 from 0x1340 and match it with 0x134 to get “308”. In addition, the DMA channel and the interrupt number have their own DIP switches. Make a note of the settings, since they will be used as parameters to the driver software in Chapter 3.

Step 9:	Reboot your computer.
If you have Windows 95 with Plug & Play support, the hardware wizard will report that it has found the PCI card. Next you will be asked if you want to install the driver or not. Be sure to choose “Do not install the driver”. The PCI’s device driver is an MS-DOS, so it is not possible to install it from the Windows 95 hardware wizard.

If the computer does not boot up properly,

- Recheck your work.

- Make sure all cables are securely plugged in

- Make sure the boards fit snugly in their slots.

- Try switching boards and slots. For instance, if your computer has multiple PCI slots, try temporarily detaching a board which is not necessary for PlayStation development. Reinstall the PCI card into that slot, and then try to start the system again. Refer to the case study in Chapter 6 to see how one person ran into similar problems and solved them.

- Read the next chapter to learn how to install the DTL-2500 device drivers and the PlayStation development software.

# Software Installation

## Before you Begin

Before following the instructions in this chapter, you should have already completed the steps in the previous chapter. This chapter describes the installation procedures for the PlayStation software tools, which are distributed in the Programmer Tools CD DTL-H2500 Update disk.

## Installing Software Drivers and Software Tools

In the following steps, we assume that the local hard drive is your “c:\” drive and your PC CD-ROM drive is “ d:\”:

Step 1:	Insert the Programmer’s Tools CDROM
Insert the Programmer’s Tools CD (DTL-S2002) into your CD-ROM drive (not the DTL-H2510) of your system. If you have Windows 95, you can run the Setup program, "setup.bat" in the root directory of the CDROM. Follow all of the instructions. After the computer reboots (to set environment variables) skip to Step 6.

Step 2:	Install the “psx” tool
The directory “[cdrom]:\psx” contains the PlayStation Development directory, which includes over 100 sample programs with full source code, the includes, and the linking libraries.

If applicable, back up or delete your previous "c:\psx" directory.

To be consistent with the automatic installation of the software under Win95, we are creating a parent directory “PS” which all PlayStation software will be installed under.  Copy the "psx" directory, d:\psx, from the CD to your local hard drive c:\ps\psx:

xcopy /s d:\psx c:\ps\psx

(or just drag and drop the folder).

- Add  the line

set path=%path%; c:\ps\psx\bin

to the end of your “c:\autoexec.bat” file.

Step 3:	Install the “pssn” tools
The directory “[cdrom]:\pssn” contains the standard PlayStation development system, which includes an interactive debugger and the C compiler.

- If applicable, back up or delete your previous "c:\pssn" directory.

- Copy the "pssn" directory, d:\pssn, from the CD to your local hard drive c:\ps\pssn.

- Copy the contents of the "gnu" directory, d:\gnu, from the CD to your local hard drive c:\ps\pssn:

- xcopy 	/s 	d:\gnu\* 	c:\ps\ pssn \bin

- Add the following line to the end of your “c:\autoexec.bat” file.

set path=%path%;c:\ps\pssn\bin

Step 4:	(Optional) Install the “psxgraph” tools.
The directory “[cdrom]:\psxgraph” contains the tools for converting between standard graphics file formats and the PlayStation formats. Although we are setting up the “Graphic Artist Tools program” area, it does not contain the entire tool set for the Graphic Artist Tools. Only the conversion tools are included on this CD. Please contact your regional tools coordinator for information on how to obtain the Graphic Artist Tools CD (DTL-S220).

- If applicable, back up or delete your previous "c:\psxgraph" directory.

- Copy the "psxgraph" directory, d:\psxgraph, from the CD to your local hard drive c:\ps\psxgraph.

- Add the following line to the end of your “c:\autoexec.bat” file.

set path=%path%;c:\ps\psxgraph\bin

- Copy the all files located in the "system" directory, d:\psxgraph\system, to the window's system directory, i.e. c:\windows\system. These files are used by the Movie Converter.

- If you have Windows 95, skip to step 5. Otherwise, you will have to create the groups and match the icons yourself, by performing the following steps in Windows 3.1:

Graphic Artist Tools program group
Create a Graphic Artist Tools program group in the Windows 3.1 environment.

- Under the Program manager "File" pulldown click on the "File>New" button.

- Select Program Group; click OK.

- Fill in the Description "Graphic Artist Tools". You may leave the "Group File" field blank. A new group will be displayed.

- You are now ready to add the individual tool icons. Please follow the individual program installation instructions listed below if you are using Windows 3.1.

Note: For additional details on setting up program icons, please refer to your Windows 3.1 manual.

Movie Converter
With the "Graphic Artist Tools" program group selected, create a program icon for the Movie Converter tool:

- Under the Program manager "File" pulldown, click on "File->New" button.

- Select Program Item and click OK.

- A Program Item Properties dialog will pop up. Type “Movie Converter” in the  Description field.

- Use Browse, to identify the name of the executable to be placed in the "Command Line" field. (i.e. c:\ps\psxgraph\bin\movconv.exe)

- Click OK.

Movie Pack
With the "Graphic Artist Tools" program group selected, create a program icon for the Movie Pack tool:

- Under the Program manager "File" pulldown, click on "File>New" button.

- Select Program Item and click OK.

- A Program Item Properties dialog will pop up. Type "Movie Pack" in the Description field.

- Use Browse, to identify the name of the executable to be placed in the "Command Line" field (i.e. c:\ps\psxgraph\bin\movpack.exe).

- Click OK.

3DStudio Plug-In
This release is for 3DStudio plug-in utilities. We highly recommend attaching the 3DStudio dongle before progressing with a modeling session utilizing the 3DStudio plug-in.
Warning: Do not remove or add dongles while the PC is powered on. Do not start a 3DStudio plug-in session before performing the following:

- Remove dexbios (only if dexbios is installed)

- Remove mess1.com (only if mess1 is installed)

- Remove cdbios (only if CDBIOS is installed)

Please read the files *.doc and *.txt in the "c:\ps\3rdParty\3ds" directory. Specific installation instructions are included in the 3dstod_e.txt file.

Step 5:	Add environment variables.
Edit your autoexec.bat file to contain the lines listed below. 
Note: This example depends on where you have set up your root PSX and Pssn directory. The file paths contain forward slashes, unlike the normal DOS convention which uses backward slashes.

REM ===== PSX Development Environment Variables =====

set SN_PATH=c:/ps/pssn/bin

set COMPILER_PATH=c:/ps/pssn/bin

set PSX_PATH=c:/ps/psx/bin

set C_INCLUDE_PATH=c:/ps/psx/include

set C_PLUS_INCLUDE_PATH=c:/ps/psx/include

set LIBRARY_PATH=c:/ps/psx/lib

set GO32=DMPISTACK 1000000

REM ===== GNU C/C++ ==================================

set GO32TMP=c:/tmp

set TMPDIR=c:/TMP

REM If your computer does not have a floating point

REM co-processor then uncomment the following line:

REM set GO32=emu c:\ps\pssn\bin\emu387

REM ==================================================

The file c:\ps\pssn\bin\SN.INI is referenced by the compiler. This file can be used to contain some of the DOS environment variables. When the environment variables and SN.INI are both defined, SN.INI is given preference. For example, your c:\ps\pssn\bin\SN.INI file could include

[ccpsx]

stdlib=libapi.lib .....

set PSYQ_PATH=c:\ps\pssn\bin

COMPILER_PATH=c:\ps\pssn\bin

LIBRARY_PATH=c:\ps\psx\lib

C_INCLUDE_PATH=c:\ps\psx\include

to achieve the same result.

Step 6:	Verify your ability to compile.
To make sure you can compile, reboot your machine to register the environment variables. Make sure your paths are set correctly. If they are not, you may have to increase the environment memory space in your config.sys, using a line similar to this:

shell = command.com /E:1024 /p

The "/E:1024' sets the environment size to 1024 (valid ranges are from 160 to 32768), and '/p' makes this command.com the default command prompt. (See p.342 of Peter Norton's Complete Guide to DOS 6.22 6th Edition for further details).

Once you are certain that the paths are set up correctly, you can proceed to compile. At an MS-DOS prompt, type the following two lines:

cd c:\ps\psx\sample\graphics\basic

psymake all

The sample should compile with no errors and return a command-line prompt. If you have problems, please recheck your steps. Otherwise please contact us (refer to the section in Chapter 1 about Technical Assistance).

However, since the device drivers for the DTL-H2500 board have not been installed yet, the program cannot be run. The following steps explain how to do this.

Step 7:	(Optional) Install the CDROM emulator software.
If you have the CD-ROM emulator, you can install it now. However, since you may encounter problems, we recommend that you defer this installation until after you have finished installing the driver software for the DLT-H2500. Finish the rest of the installation and then return to this step.

Read the "readme.txt" that came with your CD-ROM emulator kit, since this is a full set of instructions for setting up your emulator card. In addition, note that the “cdbios” driver contains commands of the following form:

cdbios /a<address> /d<dma>  /i<interrupt>

The address, dma channel, and interrupt number correspond to the three DIP switch settings on the ISA board. Although the emulator board’s actual address is in 4 byte hexadecimals, the DIP switch host’s A15-A4 3 bytes are in decimal format. The actual addresses and a table of their equivalents are entered below:

Decimal	Hex	Actual	Remarks

Notation	Notation	Address

(in hex)

300	0x12C	0x12C0

308	0x134	0x1340	Default

310	0x136	0x1360

318	0x13E	0x13E0

380	0x17C	0x17C0

388	0x184	0x1840

390	0x186	0x1860

398	0x18E	0x18E0

In this case, take A15-A4 from 0x1340 and match it with 0x134 to get “308”. For more information, please refer to the “CD Emulator” book on the Technnical Reference CDROM.

In some PC’s it is difficult to use the short ribbon cable supplied with the emulator board to the DTL-H2500. The cable is standard and a longer version can be acquired from most electronic shops.

Step 8:	Install the device drivers for the DTL-H2500.
You are now ready to read the next section, "Installing the H25DRV.EXE driver."

## Installing the H25BIOS.COM driver

Now you can install the SDevTC driver H25BIOS.COM. This driver allows you to use the SDevTC tools run, pqbload, resetps, and DBUGPSX version 4.93 (or above).

Step 1:	Run FLASH.BAT.
Your PCI board has a flash ROM. Install the new kernel and debug stub into the flash ROM by running FLASH.BAT. This needs to be done only once.

cd c:\ps\pssn\bin\DTLH2500\H25BIOS

flash.bat

Step 3:	Invoke the H2500 device drivers for the Psy-Q tools.
Type the following in an MS-DOS prompt:

h25bios.com

mess1.com

Unlike the previous H2000 drivers, these are effective only for the DOS-session in which these commands are executed. We do not recommend putting these in your “config.sys” or “autoexec.bat”, so you must run these device drivers after every reboot. Under Windows 95 you may want to put these in a .BAT batch file associated with a DOS box used for development.

You can turn off the “h25bios.com” by typing

h25bios.com

Note to PlayStation developers familiar with the DTL-H2000 ISA Developer’s Board: You should now be able to use PQBLOAD, RUN, RESETPS, DBUGPSX as you normally did using the ISA board. There is no need to RUN SNPATCH.CPE. You can switch from Emulator to CD-ROM drive by running SELCD.CPE, SELEMU.CPE and using CDEXEC.CPE as with DTL-H2000 systems. Use TESTMESS for message output.

Step 4:	Run a sample program.
To verify that the installation was successful, run a sample program such as “main.cpe”. Type the following lines:

cd c:\ps\psx\samples\graphics\basic

psymake all

resetps

run main.cpe

A menu should appear on a blue screen. If you have problems running the program, please recheck you work and read the chapter “Troubleshooting the installation”.

For some of the other samples in the “c:\ps\psx\sample” directory, you must execute “psymake load” to load the data into the RAM of the PCI card. Read the individual “readme.txt” files in each sample subdirectory to learn more about the samples.

Note the following syntax for the functions "h25bios" and "resetps":

H25BIOS (Version 1.36 or above)

Usage : h25bios <options>

Options :

/b <size> 	; specify file transfer buffer size (in K bytes 2 - 32)

/p		; set PAL video mode (NTSC is default)

RESETPS (Version 1.04 or above)

Usage: RESETPS <optional switches> <num> <optional switches>

Switches for DTL-H2500 only:-

/n	set NTSC video mode

/p	set PAL video mode

Step 5:	Run other sample programs.
Programs can be built by giving the command PSYMAKE. The makefile can also be used to run a program as some programs the preloading of model and texture data before being executed.

For some of the samples you may need to execute psymake load to download the necessary data files to the development boards.

The following is a list of file suffixes which may be found in some of the sample directories:

.c	C source

.h	C include (header) file

.obj	object file

.sym	symbol file

.cpe	PS-X executable file

.tim	texture data file

.tmd	3D model data file

.lnk	psylink command file

makefile.mak	makefile for building executable

All of the samples assume that the "pssn" and "psx" directories were placed directly in the "c\" directory. If you have a different directory structure for the PSX libraries and header files, you will need to modify the .lnk files for some programs. The .lnk linker command file specifies the file path where the libraries can be found and additional object modules used in the program.

## Running from the CDROM

No extra software drivers need to be installed to run the external CDROM drive DTL-H2510 (the "black box") other than the usual H25DRV.EXE or H25BIOS.COM.

#### Running with H25BIOS.COM

- Place a PlayStation bootable CD containing your favorite game into the DTL-H2510.

- Type the following at a DOS-console prompt.

resetps 1

run /w5 c:\ps\pssn\bin\selcd

run /w5 c:\ps\pssn\bin\cdexec

- Have fun!

Note that the gold disk can be made bootable by following the instructions in the document "faq\cd4.pdf" on the Technical Reference CD.

## Debugging using the DTL-H2500

#### Debugging with Printf

If you sprinkle "printf's" throughout your code, the output will be directed to your MS-DOS window.  Here is the entire procedure to follow:

- Make sure that h25drv.exe is not running.

- At an MS-DOS command prompt, type "h25bios.com" the PSSN driver.

- Type "mess1.com" to start the message handler TSR.

- Type "resetps" to reset the h2500 board.

- (Optional) If you need to run from the cd, type "run c:\ps\pssn\bin\selcd".

- (Optional) If you need to run from the emulator, type "run c:\ps\pssn\bin\selemu".

- Run your program.

- Type "testmess" to see your messages.

#### Debugging with dbugpsx

Alternatively, you can use "dbugpsx". This is described in detail in the manual "Psy-Q Development Environment".

- Compile your source files with the “-g” option enabled (as in “ccpsx -g main.c -o main.cpe,main.sym”).

- Make sure the h25bios.com driver is running. Type “ Resetps”. Make sure the SN security dongle is installed on your printer port.

- At an MS-DOS prompt, type “dbugpsx foo /e”.

- This should bring up the blue debugging window.

## Miscellaneous

Compiler 
For a quick summary on the compiler, please refer to the ccpsx.pdf document included in the compiler document on the Technical Reference CD (DTL-S2002), in the directory progcd\gnu\doc.

The GNU CC document is also available.

No Floating Point Co-processor on PC
If your PC does not have a floating point co-processor then add the following line in your autoexec.bat as well.

set GO32=emu c:\ps\pssn\bin\emu387

Debugger
For a quick tutorial on how to use the debugger, refer to the file debugdoc.txt in the Technical Reference CD, in the progcd\pssn\debugger directory.

# Troubleshooting the installation

## Preliminaries

Check your board. It should have at least a PD2 chip on it.

Check your interrupts. Your PCI card should be assigned to one interrupt, and no other peripherals can be assigned to that interrupt. You can use the "MSD" program included in most versions of DOS. Alternatively, in Windows 95, you can see what interrupts are assigned by performing the following:

Go the “My Computer” icon. Yours may be named differently, but it looks like a computer:

- Right click on the icon and select “Properties”.

- Select the “Device Manager” tab.

- Select the “Print” button.

- Choose the "All devices and system summary".

- Print out the document.  For available IRQ's, read the "IRQ SUMMARY" section.

Alternatively, some IBM PC-Compatibles are equipped with a "Setup" routine hidden in the boot sector of the boot-up hard drive which can be accessed during a cold-boot (turning off the computer's power supply, then turning it back on). After the computer runs its memory check and the cursor moves to the top-right corner of the screen, you can hit a function key (F1 through F10) to get into the "setup" mode. Since computers vary, you may have try them one at a time. If you have a manual for your computer, read it for more information.

We are working on improving the PCI card so that it will not have this problem in the future.

## Problem: PC fails to recognize board

The DTL-H2500 main board is the board for the PCI slot. When a PC is started up with the main board mounted into the slot, the PC usually recognizes the main board automatically. However, some models of PCs may fail to recognize it. The problem with the PCI BUS interface can be caused by BIOS bugs or hardware bugs. Currently, there are no general purpose solutions for either bug. PC models which may cause problems should not be used. Please check the table at the end of this chapter.

## Known problems with BIOS of PC

Problem	Workaround

AMIBIOS Intel Endeavour version 1005C B0	Use AMIBIOS Version 1002CB0

## Diagnostic steps

If your board is not in the tables below, in the section “General known compatibility …”, then the board's addresses could be incorrect, or the PCI bus is malfunctioning. Before you can proceed with the diagnosis, the H25DRV.EXE must be installed, and the BIOS settings may need to be changed. These are explained in Step 1 and Step 2 below.

Step 1:	Install the H25DRV.EXE driver.
Before the DTL-H2500 main board can be used, the DTL-H2500 main board should be mounted into the PCI slot (see the chapter “Installing the Hardware”), and the device driver for DTL-H2500 should be installed as well (see the chapter “Installing the Software”, in the section “Installing the H25DRV.EXE driver). Make sure that you do not have a dexbios or h25bios.com running.

Step 2:	Change the BIOS settings.
The BIOS settings may need to be changed for some models of PCs. A setup utility is usually accessible on PC’s by hitting the F10 key:

- Turn off your computer.

- Turn on your computer. A memory check will be run.

- When the memory check finishes, the cursor will appear on the right of the screen. Hit the F10 key to invoke the CMOS setup program.

The main changes to be made to the settings are as follows:

- Enable the PCI BUS

- Disable the Shadow area in RAM

- Disable the 4 KB Shadow area to be allocated to the DTL-H2500.

Step 3:	Check for failure in address mapping.
Executing FRESET.EXE informs you which addresses the PC allocates for the PCI card. Generally, the PC allocates an area with one of addresses 0x000C8000 to 0x000EFFFF.

C:\ps\psx\bin\FRESET.EXE

and the following message should appear:

PCI version 2.10 Special Cycle 1, Config Mechanism 1 1 bus

bus 0: dev=13 func=0 irq=11, io=000C8000

UNIT 0: I/O addr=0x000C8000, IRQ=11(vect=0x0073, 8259=a0)

If “addr” is set to any of the addresses outside of the range from 0x00000001 to 0x000FFFFF (1 M or less), the address mapping by the PCI will fail.

The solution is to modify the mapping with a special utility for modifying the address mapping called REALLOC.EXE. Typing the following will allocate some PCI memory at 0x000c8000

realloc 0x000c8000

When the address has been modified, warm-boot the PC (with Alt+Ctrl+Del). The address should be mapped to 0x000c8000, and the main board should begin to work properly. This method has been shown to work for DEC PCs (DECpc XL 466d2 and DECpc LPx 560).

Step 4:	If the previous step did not work, check for problems with the PCI bus interface.
If the address mapping succeeds, check the PCI interface operation with the DOS debug command by doing the following:

Type “debug”. You are now in the debugging mode of DOS.

At the “-” prompt, type “d c800:0000”. This number is the lead address of the area allocated to the PCI board.

You should see something similar to the following:

C800:0000  01 01 00 09 01 01 00 09-01 01 00 09 01 01 00 09   ................

C800:0010  01 01 00 09 01 01 00 09-01 01 00 09 01 01 00 09   ................

If the PCI works properly, a large number of numeric characters are displayed with regular patterns by 4 bytes. If numeric characters are displayed with an  irregular pattern, there is a problem with the PCI BUS interface.

## Problem: Board will not reset/Can’t run programs

Step 1:	If using decicons, and the boards do not reset correctly (F9, F10) or download programs (F4): Try reinstalling the PlayStation OS ROM image on the board, using Flashb8.bat. (Remember not to use PFLASH.BAT if you wish to use DECICONS).

Step 2:	If using decicons, try running decicons on your Win95 PC in “DOS” mode, rather than in a DOS shell. If this works, it would imply that you have a Win95-related clash.

Step 3:	If step (2) did not help (or you are not using decicons), use the standard DOS debug option to examine the memory used by the driver. i.e. if 0xE7000 is the start of the address space allocated for the driver, run debug, and type “-d e800:0”. A hex dump will be displayed. If the sequence is not of the form “01 00 09 00 01 00 09 00-01 00 09 00 01 00 09 00”, then review the "shadow" setting of BIOS. The "shadow" at E7000h (in this case) must be disabled.

Step 4:	If this does not cure the problem, check the version of BIOS that you are using. SCEE found problems with some H2500’s in PC’s with AMI BIOS and Intel mother boards. In particular, BIOS version 1.00.02.CB0 works, but later versions do not. Their solution was to downgrade the PC’s BIOS to 1.00.02.CB0 and reinstall Win95. (Win95’s installer appears to make some choices about the hardware and these cannot be changed once the program in installed.) This file can be downloaded from:

ftp://ftp.funet.fi/pub/hw/vendors/intel/bios

There is a known problem with this version of the BIOS in that sometimes it does not recognize the PC hard-disk. Rebooting with CTRL+ALT+Delete seems to cure the problem. We will let you know when we find the latest version of the BIOS which works correctly.

## General known compatibility issues with PC Brands and models

The following table lists known computer compatibility issues with the DTL-H2500 board. It consists of lists compiled from SCEI, SCEE, SCEA, and

licensees. This list was recently updated with a new set of information in August 1997. Note that, although the boards may have worked for

the computers here, you may have differing levels of success. This list is not a guarantee that the boards will work for all instances of these

computers. Contact Technical Support if you have further difficulties. We use Compaq computers, and do not recommend Dell or Gateway because

they tend to use a custom BIOS that makes installation very painful.

| Brand | Assus | Archipelego |  | AST | Compaq | Compaq | Compaq |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Model | Tech P/I - P55TP4N / 100 | P133 |  | Manhattan V Series 5090 | Deskpro 575 | Deskpro 5100 | Deskpro 5166 |
| Cpu | - | - |  | Pentium 90 MHz | Pentium 75 MHz | Pentium 100 MHz | Pentium 166 MHz |
| BIOS | Award Modula Bios V.4.51 PG from Award Software | - |  | AST Manhattan BIOS Rel 1.12 | - | - | - |
| Motherboard | P166 | - |  | - | - | - | - |
| Slot | - | - |  | - | - | - | - |
| HDD | - | - |  | - | - | - | - |
| Video | - | - |  | - | - | - | - |
| Memory | - | - |  | - | - | - | - |
| PCI Chipset | - | Intel Endeavour |  | - | Compaq | Compaq | Compaq |
| OS | - | - |  | Windows 95 | Windows 95 | Windows 95 | Windows 95 |
| Problem | OK | OK |  | OK, but see warnings below | OK , but see solution below | OK , but see solution below | OK , but see solution below |
| Solution | - | - | You will need to try the PCI board at different PCI slots. If your machine does not finish booting properly, try the next slot. If H25DRV.EXE does not work, skip to the section "Installing the H25Bios.com Driver" |  | You can assign the PCI interrupts by invoking the Compaq setup utility at boot up time. Cold boot the machine, then hit F10 after the memory test finishes. Navigate to the main screen and select the "Add-In Devices" screen. You can thereby change PCI interrupts. |  |  |

| Brand | Compaq | Compaq |
| --- | --- | --- |
| Model | Deskpro 6000 | Compaq Deskpro 6200 |
| Cpu | Pentum 200 MHz | Pentium Pro 200MHz |
| BIOS | - | Unidentified (specifically designed by Compaq) |
| Motherboard | - | PCI 1-Slot + ISA 1-Slot + PCI/ISA 1-Slot |
| Slot | - | - |
| HDD | - | - |
| Video | - | Matrox MGA Millennium 4MB |
| Memory | - | 16MB x 2 (SIMM Socket x 8) |
| PCI Chipset |  |  |
| OS | Windows 95 | "Windows NT 3.51 Workstation" and "Windows 95" coexisted. Both OSs were set bootable. |
| Problem | OK, but see Solution below. | The unit operated correctly with factory setting, after following the steps presented in "Solution" below. |
| Solution | You can assign the PCI interrupts by invoking the Compaq setup utility at boot up time. Cold boot the machine, then hit F10 after the memory test finishes. Navigate to the main screen, and select the "Add-In Devices" screen. You can thereby change PCI interrupts. | Step 1. Prior to connecting H2500, start BIOS SETUP(DIAG) once. Check that no message from SETUP, such as "The boards have been detected/deleted" is displayed at this point. Step 2. Check that the boards are recognized by SETUP. If they are, select "Save and Exit". Step 3. Power off the unit and connect H2500. Start BIOS SETUP once again. Check that the message, "The new board has been detected", is displayed. H2500 must have been recognized as "Other Device". You can find it from the description, "SONY", at the Inquiry indicator. Check that IRQ and Memory are allocated. If they are, select "Save and Exit". (The actual figures were 4 for IRQ and C8000-C8FFF for Memory.) Step 4. Start Windows 95. (It may be better to power off once before starting Windows 95.) Check that the new board has been detected. Then at driver installation select "Without Driver". After executing H25BIOS from DOS Prompt if Color-Bar gets displayed, it is no problem. You may make sure of it with "System Property" just in case. Step 5 (Optional) If Color-Bar is not displayed at this point, you need to do the set-up again from the first after following the steps below: A. Delete H2500 at "System Property" and power off. B. Remove H2500 and restart Windows 95. Check that H2500 has been deleted with "System Property" (The message from BIOS SETUP, "The board has been deleted", must be displayed.) |

| Brand | DEC | DEC | DEC | DEC | DEC | DEC | DELL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Model | DEC Personal Workstation 200i | Celebris FP590 | Celebris GL5133ST | Celebris GL6200ST | Celebris XL5100 | Venturis FP5100 | MT XPS |
| Cpu | Pentium Pro 200MHz | - | - | Pentium Pro 200 MHz | - | - | Pentium Pro 200N |
| BIOS | Phoenix ROM BIOS 4.05 | - | - | - | - | - | - |
| Motherboard | (specifically designed by DEC) | - | - | - | - | - | - |
| Slot | PCI 2-Slot + PCI/ISA 3-Slot | - | - | - | - | - | - |
| HDD | 2.1GB(SCSI) | - | - | - | - | - | - |
| Video | AccelGraphics AccelR8 | - | - | - | - | - | - |
| Memory | 96MB [32x2 + 16x2] (SIMM Socket x 8) | - | - | - | - | - | - |
| PCI Chipset | - | - | Triton |  | Neptune | - | - |
| OS | "Windows 95 " only | - | - | - | - | - | - |
| Problem | The unit operated correctly with the factory setting. However, for the set-up you need to follow the steps below in "Solution". | OK | OK | OK | OK | OK | OK |
| Solution | Connect H2500 to the second Connector from the uppermost. (Usually, Video Card is connected here.) If you connect H2500 to a connector other than that above, you will see a message informing you that it has caused a conflict with the Motherboard Resource, and the unit does not operate correctly | - | - | - | - | - | - |

| Brand | DELL | DELL | DELL | DELL | DELL | DELL | DELL | DELL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | OptiPlex GXPro 200 | OptiPlex GXi 5200M | Optiplex GXM 5200 | P120T Pentium | XM5100 | XMT5100 | XMT5120 | XPS P120C |
| Cpu | Pentium Pro 200MHz | Pentium 200MHz | 200 MHz | - | - | - | - | - |
| BIOS | Phoenix ROM BIOS PLUS 1.10.A02 | Phoenix ROM BIOS PLUS 1.10.A00 | - | - | - | - | - | - |
| Motherboard | (specifically designed by DELL) | (specifically designed by DELL) | - | - | - | - | - | Specifically designed by Dell |
| Slot | PCI 3-Slot + PCI/ISA 2-Slot | PCI 2-Slot + ISA 2-Slot + PCI/ISA 1-Slot | - | - | - | - | - | - |
| HDD | 2 + 2GB(IDE) | 3.0GB(IDE) | - | - | - | - | - | - |
| Video | #9 Imagine128 | S3 Trio64V+ | - | - | - | - | - | - |
| Memory | 64MB EDO x 1 (DIMM Socket x 4) | 96MB SDRAM [64x1 + 32x1] (DIMM Socket x 4) | - | - | - | - | - | - |
| PCI Chipset | - | - | - | - | Neptune | Neptune | Neptune | - |
| OS | "Windows NT 4.0 Workstation" (pre-installed) overwritten by "Windows 95" (Only "Windows 95" was set bootable.) | "Windows 95 (OSR2)" only (pre-installed) | - | - | - | - | - | Windows 95 |
| Problem | The unit operated correctly with the factory setting. However, for the set-up you need to follow the steps below. (There may be difficulties in modifying the interrupts.) | The unit operated correctly with factory setting. (There may difficulties in modifying the interrupts.) | Incompatible. (There are unsolvable difficulties in modifying the interrupts. ) | Incompatible? | OK | OK | OK | Incompatible |
| Solution | Connect H2500 to one of the two PCI Connectors in the lowest part (either PCI1 or PCI2). If you connect H2500 to other connector than those above you will see the error messages, "Plug & Play Configuration Error" and "Strike the F1 key to continue, F2 to run the setup utility", and the unit does not operate correctly. | Not applicable. | - | - | - | - | - | Do not use! |

| Brand | FMV | Fujitsu | Gateway | GoodTech | HotChips | HP | IBM | IBM | IBM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model | - | FMV 590DE | Gateway 2000 P5-200 | P200 | Pentium Pro 200 | Vectra XM | PC 750 | PS/V Master 100 | PS/V Master P120 |
| Cpu | Pentium Pro 200 MHz | - | Pentium 200 MHz | - | - | - | - | - | - |
| BIOS | - | - | AMI 1.00.06.CY1T | - | - | - | - | - | - |
| Motherboard | - | - | Intel ATX Motherboard | - | - | - | - | - | - |
| Slot | - | - | PCI 3-Slot + ISA 2-Slot + PCI/ISA 1-Slot | - | - | - | - | - | - |
| HDD | - | - | - | - | - | - | - | - | - |
| Video | - | - | - | - | - | - | - | - | - |
| Memory | - | - | - | - | - | - | - | - | - |
| PCI Chipset | - | - |  | - | - | VLSI | - | - | Triton |
| OS | - | - | "Windows 95" only (pre-installed) | - | - |  | - |  | - |
| Problem | OK | OK | The unit operated correctly with the factory setting when used as stand alone. However, with Network Card or SCSI Card installed in the unit, it hung up after executing h25bios. | OK | OK | OK | OK | OK | Incompatible |
| Solution | - | - | Set either "Standard" or "OFF" at Power Management setting in Windows 95 Control Panel. ("Details" is set as default setting.) | - | - | - | - | - | - |

| Brand | Mesh | Micron | Micron | Micron |
| --- | --- | --- | --- | --- |
| Model | Elite 120R Pentium | Client D (MARL-P200-MT) | Millennia D (M55HIPLUS-P200-MT) | Millennia Plus |
| Cpu | - | Pentium 200MHz | Pentium 200MHz | Pentium 200 MHz |
| BIOS | - | AMI 1.00.07.DB05 | Phoenix 4.05 | - |
| Motherboard | - | (Intel ATX Motherboard) | (MICRONICS Motherboard) | - |
| Slot | - | PCI 3-Slot + ISA 2-Slot + PCI/ISA 1-Slot | PCI 3-Slot + ISA 3-Slot + PCI/ISA 1-Slot | - |
| HDD | - | 2.1GB(IDE) | 3.1GB(IDE) | - |
| Video | - | #9 9FX Motion 2MB EDO DRAM | #9 Imagine128 4MB VRAM | - |
| Memory | - | 8MB EDO x 2 (SIMM Socket x 4) | 16MB EDO x 2 (SIMM Socket x 4) | - |
| PCI Chipset | - | - | - | - |
| OS | - | "Windows 95" only (pre-installed) | "Windows 95" only (pre-installed) | - |
| Status | Incompatible | The unit did not operate correctly with the default setting. Plug & Play failed and Color-Bar was displayed. | The unit did not operate with factory setting. | Incompatible |
| Solution | - | Clear the check of auto setting and manually reset "Memory Range" at System Device Manager in Windows 95 Control Panel. (e.g. : Memory Range is D0000-D0FFF) | Set "OFF" at Plug & Play OS setting in BIOS setting. | - |

| Brand | Micron | Olivetti | Oki |
| --- | --- | --- | --- |
| Model | Millenia Pro A (LIGHT-PP200-MT) | M4 82 | IF Station 590/DG |
| Cpu | Pentium Pro 200MHz | - | - |
| BIOS | Phoenix 4.05 | - | - |
| Motherboard | (MICRONICS Motherboard) | - | - |
| Slot | PCI 3-Slot + ISA 1-Slot + PCI/ISA 1-Slot | - | -- |
| HDD | 3.1GB(IDE) | - | - |
| Video | #9 Imagine128 | - | - |
| Memory | 32MB EDO x 1 (DIMM Socket x 4) | - | - |
| PCI Chipset | - | - | - |
| OS | "Windows NT 3.51 Workstation" and "Windows 95" coexisted (Both Os's were set bootable.) | - | - |
| Problem | The unit operated correctly with the factory setting for the reason described below. | Incompatible | OK |
| Solution | The unit operated correctly with the factory setting, because the factory setting of the Plug & Play OS setting in the BIOS was set "OFF". | - | - |

# DTL-H2500 Questionnaire

We would greatly value your help with the attached questionnaire so that we can let other developers know about other problematic PCs.

| DTL-H2500 QUESTIONNAIRE Please FAX back to (Attention: Developer Support) at: 44 (0) 171 390 4324 (Europe) 1-650-655-5511 (United States) |  |  |  |
| --- | --- | --- | --- |
| Company Name | Contact Name | Telephone Number |  |
| DTL-H2500 Serial No. |  |  |  |
| Is your system in use ? If yes, for how long ? |  |  |  |
| Make of PC and OS (e.g. Archipelego, Win95) |  |  |  |
| PC Speed and RAM (e.g. 120MHz, 16Mb) |  |  |  |
| PC Chipset (e.g. Triton) |  |  |  |
| PC BIOS Manufacturer/Version (e.g. AMIBIOS 1.00.02.CB0) |  |  |  |

# Case study

Since every IBM-PC compatible seems to have its own character, you may have to take additional steps during the installation. For instance, you may have to move IRQs around, or switch boards and slots. Here is the experience and advice of one of our techies, who was trying to install a DTL-H2500 PCI board and the CDROM-emulator card.

The PSX development system using the DTL-H2500 PCI development board will need two IRQs for its exclusive use, one for the PCI Dev. board and the other for the CDROM emulator. The PCI Dev. board tries to use IRQ11, so it will be safe to have this interrupt free for its exclusive use. The emulation card uses one of the IRQ 5,7,10,11,12,15.

Before we begin installing the development boards on a Windows95-based system, we should follow the following steps :

Step 1:	Switch on your machine and find out the list of IRQs, DMA Channels and I/O addresses already in use. If you are using Windows95 the process is very easy. Choose "System" from the control panel and click on the Device Manager. Printing "All devices and system summary" will make a hard copy of all various system resources utilized and various drivers loaded.

Step 2:	Install the PCI Dev. Board in one of the empty slots and boot up your machine again. Go into the device manager and check which IRQ has been assigned to our Dev. Board. If the IRQ assigned to the Dev. Board is available for its exclusive use (i.e. is not shared by any other devices), you are all set. The system generally assigns IRQ11 to the Dev. Board.

Otherwise you should shut down your machine and remove the Dev. Board. Boot up your machine and go into BIOS setup by pressing F10 (in most cases) while your machine is just starting up. Try to reassign IRQs to various devices and free IRQ11 (or the one the Dev. board was requesting earlier in this step). Save the BIOS setting and quit the system setup. Boot up your machine. Under Windows95 you should be able to check the list of IRQs in the device manager. The IRQ11 should be free at this point. If any other device is trying to use that IRQ, disable it under the device manager.

While reassigning IRQs under system setup, also try to free one of the IRQ 5,7,10,12,15 for the CDROM Emulator card.

Step 3:	Shut down your machine once again and install the PCI Dev. board. Rebooting your machine will assign IRQ11 to the PCI development board. Under Windows95, you can go back into the Device Manager to verify that IRQ11 is being used exclusively by the development board only. At this point you should have one of the other IRQs which you freed in the previous step for Emulator use.

Step 4:	Follow the steps given in the Installation Guide to install various drivers and test to see if your system is working properly.

Step 5:	Shutdown your machine. Change the jumper settings on the CDROM Emulator board to reflect the IRQ (freed in Step 2 above), the DMA channel and the IO addresses. Put the card in one of the empty slots and connect it to the PCI Development board using the 10-pin flat cable. Connect the emulator drive (follow the steps given in the CD Emulation manual). Reboot your machine and try installing various drivers and emulator initialization routines.

Step 6:	The system is now ready for PSX development.

Another tech supporter discovered that the PCI board was being assigned to IRQ 9, which was shared by other devices. Again, since your computer and its configuration could be different from our own, your mileage may vary.

# Installing to an AMI BIOS Machine

Machines using AMI BIOS (1.00.05.CB0,1.00.06.CBO) may cause problems when you are installing the DTL-H2500. Follow these steps:

Step 1:	"Uninstall" the PCI card.

- Go the “My Computer” icon. Yours may be named differently, but it looks like a computer:

- Right click on it; select “Properties”.

- Select the "Device Manager" tab (see figure below).

- Double-click on "Other Devices", then select "PCI card" (circled in the figure below).

- Select "Remove".

Step 2:	Shutdown Windows 95.

Step 3:	Clear the NV-RAM of the motherboard.

Step 4:	Reboot the machine.
When Win95 displays the message "New Hardware Found", select "Do not install a driver".

Step 5:	Change the memory location of the PCI Card Resource.
The operating system, under the AMI BIOS, will allocate the PCI card to an unsuitable memory space. This needs to be changed by performing the following:

- Open the "Device manager" as in Step 1.

- Double-click on the PCI card icon (ignore the warning icon). If the icon does not appear, reboot the machine, so that the Windows 95 operating system can install the icon. Again, if you are asked to install a device driver, choose NOT to install one.

- Select the resources tab, which will show the memory range of E7000-E7FFF.

- Clear the "Automatic settings" check box.

- Change the memory to a non-conflicting memory address, such as D4000:

- After modification, Win95 asks you "Do you want to restart now?". Select "Yes".

Step 6:	Verify that the PCI card uses the addresses you specified.
After Win95 is restarted, open the "PCI Card Resources" as before, and check to see that the Memory Range has been changed to the address  you specified. Again ignore the warning icon.

At this stage, issuing the "freset" from the DOS prompt will show that the H2500 is allocated to C8000 and at the same time, it is reset. When the PCI Card is once allocated correctly as above, it will be kept allocated to the same Memory Range (C8000-C8FFF for this example), even after rebooting with the PCI card resources being set to "automatic settings".

# Commands for the Flash-ROM

“flash.bat”, “flashb8.bat”, and the “pflash.bat” are simple batch files which call more primitive commands to the DTL-H2500; therefore, it is necessary to run the batch files in the same directory which contains “freset”, “fload”, and “fquit”. You will almost never need to know what the parameters do; but if you are curious, here is the information.

flash.bat.
A batch file to write the OS binary image into the flash ROM. It uses the file 'h2500.img' and is equivalent to typing the commands shown below in an MS-DOS prompt:

freset -r 1

fload h2500.img

fquit

freset 1

freset.
Resets the boot mode and the DTL-H2500 video mode.

Syntax

freset [-r] [-p|-n] [switch(hex)]

where

-r	The main board is booted by EPROM.
(Default booted by flash ROM.)

-n	video mode is set to NTSC mode

-p	video mode is set to PAL mode
if both are not specified, the previous mode is kept.

The switch option can have the following values:

0	boot PlayStation CD-ROM

1	wait to load files
If no switch is specified, the previous mode continues.

Fload.
Load a file to the main board and run that program. This should be done after the main board has been set to wait for loading.

Syntax

fload [-l|-r|-b<address>] file_name

where

file_name	loaded file name

-l	only loads a file

-r 	only runs without loading a file
	if both are not specified, load a file and run.

-b	<address>

loads a binary file to the specified address

fquit.
Causes the main board to quit waiting mode.

Syntax

fquit